#### Document Control

The Highways and Infrastructure Manager is the owner of this document and is responsible for ensuring it is reviewed.

| Document Reference: |  |
| --- | --- |
| Version: | 11 |
| Issuer: | Stephen Green |
| Directorate & Service | Regeneration and Environment |
| Issue Date: | September 2023 |
| Review Date: | Aug 2024 |
| Reviewers: | Dave Carr / Stephen Green |
| Audience: |  |
| Availability: | On website |
| Approval: | Lead Member for Area Management |

# Change History

| Version | Date | Issuer/Amender | Detail | Approval if required |
| --- | --- | --- | --- | --- |
| 10 | September 2022 | Dave Carr / Stephen Green | Annual Update | Lead Member for Area Management |
| 11 | August 2023 | Dave Carr / Stephen Green | Annual Update | Lead Member for Area Management |

WINTER MAINTENANCE SERVICE POLICY AND OPERATIONAL PLAN

Contents

- Statement of Policies and Responsibilities

  - Introduction

  - Policies and objectives

  - Client and Service Provider risks and responsibilities

  - Partnership or shared risks and responsibilities

  - Decision making process and responsibilities

  - Precautionary Treatments

  - Spread rates for precautionary treatments

  - Post treatments

  - Monitoring

  - Liaison and communication arrangements with other authorities and other 		Public services

  - Winter risk period

  - Resilience standard

  - Legislative background.

- Route Planning for Carriageways, Footways and Cycle Routes

  - Carriageway routes by risk level

  - Response and treatment times for all carriageway treatments

  - Routes for footbridges, subways, and other high risk pedestrian areas

  - Response and treatment times for footway and cycle route treatments

  - Routes for other footway and cycle route treatment by risk level

  - Allocation of plant, vehicles, equipment, and materials to routes

  - Location and maintenance of salt bins and grit heaps

  - Special sites or features (e.g., near railways or traffic calming)

- Weather Prediction and Information.

  - The decision-making process

  - Road weather information bureau service

  - Road weather stations

  - Timing and circulation of information

  - Road weather forecast

  - Reporting procedure

  - Thermal mapping

  - Maintenance of ice detection equipment

  - Information to be provided.

- Organisational Arrangements and Personnel

  - Command, control, and operational organisation

  - Arrangements with other authorities

  - Arrangements with other public services

  - Decision making

  - Operational record keeping and reporting

  - Plant and vehicle manning arrangements, including management of drivers’ hours

regulations

  - Materials management

  - Training and development arrangements

  - Schedules of Contract and Voluntary Personnel (CVP)

  - Employee roles and responsibilities

  - Contact and commissioning arrangements for CVP

  - Employee duty schedules, rotas, and standby arrangements

  - Winter Service exercising arrangements

  - Standard operating procedures

  - Escalation and emergency operating procedures

  - Operational monitoring

  - Health and safety procedures

  - Contingency arrangements.

- Facilities, Plant, Vehicles and Equipment

  - Winter Service compounds and facilities

  - Calibration procedures

  - Fleet inventory including licence requirements and capacity

  - Fuel stocks and locations

  - Location of plant, vehicles, snow-blowers, and other equipment

  - Contingency arrangements

  - Garaging, servicing, and maintenance arrangements

  - Contact and hire arrangements for contract plant.

- Salt and Other De-Icing Materials

  - Location and capacity of stocks for salt and other materials

  - Contacts and purchasing arrangements for supplies

  - Minimum pre-season and in-season stock levels

  - In season re-stocking arrangements

  - Testing arrangements

  - Stock level monitoring and forecasting procedures

  - Loading arrangements

  - Treatment requirements including spread rates.

  - Road surface wetness

  - Traffic levels

  - Precautionary treatment matrix

  - Determining spreading capability

  - Spread rates of salt for precautionary treatments

  - Treatments for snow and ice

  - Preparation before snow and ice

  - Depths of snow

  - Precautionary treatments before snow and ice

  - Treatments during snowfall

  - Treatments when slush is on the road

  - Treatment when thin layers of ice have formed

  - Treatment for thicker layers of ice or compacted snow

- Operational Communications

  - Technical systems information

  - Reporting arrangements and protocols

  - Inventory and allocation, including back up.

- Contingency Plan

  - Contingency arrangements for Winter Service delivery such as salt supply,

Drivers, fuel vehicles etc

  - Arrangements for implementing minimum winter networks

  - Mutual Aid e.g., resources available from adjacent authorities

  - Liaison with Category 1 and Category 2 responders (reference Civil

Contingencies Act 2004).

- Information and Publicity

  - Local press and broadcast contact information

  - Public information leaflets

  - Other key local and national contact information

  - Thermal mapping

  - Responsibilities and guidance for providing information

  - The decision-making process

  - Road weather stations

  - Information to be provided

  - Road weather information bureau service

  - Timing and circulation of information

  - Road weather forecast

  - Notification arrangements for failure to maintain the published network

  - Reporting procedure

  - Maintenance of ice detection equipment.

- Quality Management

  - Quality management regime

  - Document control procedures

  - Distribution of documents

  - Information recording and analysis

  - Arrangements for performance monitoring, audit and updating

  - Procedure for deviation from the Winter Service Plan.

  - Service review following significant events and at end of season

- Statement of Policies and Responsibilities

  - Introduction.

    - This Winter Maintenance Service Plan and Policy statement are based on the key principles for delivering best value in highway maintenance as recommended in the Department for Transport document –`Well-Managed Highways and Infrastructure Code of Practice’ published October 2016 and implemented in October 2018, incorporating the recommendations of the Institute of Highway Engineers National Winter Service Research Group.

    - The Council has a statutory duty under Section 41(1A) of the Highways Act 1980 to maintain the highway. The introduction on 31st October 2003 of Section 111 of the Railways and Transport Safety Act 2003 extended that statutory duty by the addition of the phrase “In particular, a highway authority is under a duty to ensure, so far as is reasonably practicable, that safe passage along a highway is not endangered by snow or ice.”

    - The Council also has a duty under Section 150 of the Highways Act 1980 to remove obstructions from the accumulation of snow from the highway.

    - The council also has a duty under Section 16 (1)(a) of the Traffic Management Act 2004 to “securing the expeditious movement of traffic on the authority's road network”

  - Policies and objectives

    - The Council’s winter policy statements are: -

“To provide a Winter Service that ensures, so far as is reasonably practicable, that pre-defined routes of roads and footways are treated in accordance with their priority on the highway network and prevailing weather conditions.”

    - The objective of this policy is to minimise the risk to safety that highways users confront in winter conditions, secure the expeditious movement of traffic on the network, and minimise disruption due to the non-availability of the highway network through ice and snow.  However, this Policy recognises that given the scale of financial and other resources involved in delivering the Winter Service, it is not practically possible to provide the service on all parts of the highway network, neither is it possible to satisfy the requirements of all those wishing to use the highway.

    - The Winter service plan is how the authority delivers the objectives of the stated overarching policy.

  - Client and Service Provider risks and responsibilities

    - The delivery of the winter maintenance service is the responsibility of South Tyneside Council as the relevant Highway Authority.

    - Those responsibilities include: -

- Preparing and updating the Winter Maintenance Service Plan and obtaining its approval.

- Appointment of Winter Service Managers to undertake the decision-making process, who will instigate treatment for the forecasted ice formation or snow in accordance with this service plan.

- The management of finance for the normal expected level of operations and the authorisation of additional finance to fund extended operations at times of prolonged serious weather.

- Monitoring the performance of the Winter Maintenance Service in both the prime task of protecting the travelling public from the dangers of icy roads and footways and that value for money is obtained.

- Identifying the risk-based criteria for the roads and footways to be treated, the response that will be maintained and the priority order for treatment to be carried out.

- Instigating action upon the occurrence of sudden ice formation or snow.

- Maintaining adequate communications systems throughout the Winter Service period.

- The supply and maintenance of vehicles, for the application of salt, and the removal of snow.

- The preparation of salting and snow ploughing routes in accordance with the laid down criteria

- Maintaining salt stocks to the level required to provide the service as detailed in this service plan.

- The provision of accommodation for the salt stocks and the operational base for the Winter Service operation.

- The provision and upkeep of an Ice Detection System, Weather Forecasting system and a data logging and archive system.

- The supply of technical advice for the Winter Service.

- Liaising with critical local services such as hospitals and major employers to ensure, as far as possible, that they are supported within the resources available.

  - Partnership or shared risks and responsibilities

    - The overall service is delivered by the Councils Director of Regeneration and Environment. Responsibility is then shared between the  Infrastructure and Transport team(Road Salting), and Community and Place (footway Salting). Day-to-day operations are devolved to the winter service duty manager and 	appropriate Streetscene  managers for roads and footways respectively.

  - Decision making process and responsibilities

    - The winter service manager is responsible for the day-to-day operational decisions regarding provision of road gritting during both dayshift and nightshift and the timeliness thereof. They will be guided in their decisions by this service plan and the winter service manager’s instruction and guidance 	manual (Appendix A). Appropriately trained officers undertake this duty on a weekly rota basis

    - The nightshift senior operative will receive the daily instruction from the duty manager by logging on to the Vaisala Manager system, disseminate the instructions to the drivers and commence gritting duties as instructed.

    - The winter service duty manager will also discuss the plan with the senior operative every night at the commencement of the shift to ensure the instructions are clearly understood.

    - The Streetscene managers are responsible for the day-to-day operational decisions regarding the provision of footway gritting services and the timeliness thereof.

    - After carefully evaluating the conditions, the Streetscene manager will establish the number of operatives available and any variance in conditions throughout their operational area. After gathering this information, they will prioritise their operational areas to ensure that they can assist as many residents as possible to go about their daily lives and safely access shops, businesses, and public transport. Consideration must also be given to elderly, disabled and disadvantaged residents so they can access supplies and services.

    - The respective winter service managers will record their daily decisions.

  - Precautionary treatments

    - These are the application of de-icers to road surfaces before the onset of freezing conditions (i.e., frost, snow, or freezing rain). The purpose of precautionary treatments is to prevent the formation of ice, or to weaken or prevent the bond of freezing rain or snow to road surfaces.

    - It is usually impractical to spread sufficient salt to melt freezing rain or more than a few millimetres of snow. Therefore, in advance of forecast snow or freezing rain, salt is spread to provide a debonding layer so that:

- Snow is more readily removed by ploughing

- Compacted snow and ice are more easily dispersed by traffic

    - It is very difficult to remove a layer of compacted snow or ice that is bonded to the road surface, so precautionary treatments are essential before heavy snowfall.

  - Spread rates for precautionary treatments

    - The majority of winter service treatments (and salt spread) in the UK are precautionary treatments in response to predicted frost conditions.

    - For precautionary treatments when frost conditions are forecast, the council have followed the guidance in “|NWSRG Section 8 Spread rates for precautionary salting”. The `Medium` traffic definition and the `Fair` spreader coverage is applied.

    - The appropriate rates to use depend upon the spreading capability, which in turn depends on the condition of the salt, spreader performance and calibration. The lower spread rates are used where the spreading capability is deemed to be ‘reasonable’, and the higher rates are used where the capability is ‘modest’.

    - As South Tyneside’s salt is stored outside and is not covered, the moisture content of the salt is difficult to manage and therefore classed as outside the `optimum` range which results in increased spread rates by 20% as per the guidance.

  - Post treatments

    - Post treatments may involve the ploughing of snow, the application of de-icers and the application of abrasives to ice and snow present on the road surface, or some combination of these.

    - Although de-icers will melt ice or snow directly, it is normally impractical to apply enough de-icer to melt all a moderately thick ice or snow layer.

    - Ploughing is the only economical, efficient, effective, and environmentally acceptable way to deal with all but very light snow.

    - Following extreme snow events snow should, if possible be removed from secondary roads by mechanical means prior to the application of salt to the cleared road surface.

  - Monitoring

    - The condition of routes should be monitored following treatment in order to confirm that the treatment has been effective. If it has not been fully effective, contingency treatments should be considered to achieve the required condition. It should be noted that both active and passive road weather sensor systems require the presence of moisture to determine either the concentration of an anti-icing chemical on the road or the freezing point temperature of the solution present on the road sensor.

  - Liaison and communication arrangements with other authorities and other public services

    - This winter service plan and the associated gritting routes will be made available to neighbouring authorities and trunk road maintenance agents that pass through the borough.

    - Regional winter managers for the North East of England from Stockton to Northumberland meet as required to share best practice and to promote inter authority coordination and cooperation.

    - Contact information is exchanged between the Authority and public transport operators to facilitate the sharing of up-to-date road condition and weather information. This will enable the various travel operators to update their customers by their preferred means.

    - The winter service managers are issued, at the start of the winter season, with the winter service manager lists and contact numbers for the other Tyne and Wear authorities

    - Throughout the winter the maintenance agents for the A19 and A184 forward their proposed treatment information to the Authorities through whose area their roads pass.

    - Liaison with other public services such as emergency services is undertaken depending upon the particular circumstances.

  - Winter risk period

    - Overall Winter Period.  The provision of weather forecasting services commences on the 1st of October and runs to 30th April.

    - A limited permanent gritter driver nightshift and activation of the winter service managers duty rota commences on or about 17th October

    - Core Winter Period. The Core winter period runs from 1st November to 31st March.

    - The full gritter driver nightshift cover commences on or about 1st November. This permanent core nightshift cover runs until 31st March when, if weather permits, the gritter driver numbers are reduced and then operate until about 14th April when conditions should allow the service to be terminated for the season.

    - Days Resilience (Overall Winter Period) 12 Days

#### Resilience standard

    - In accordance with current guidance, we work to the minimum resilience recommended in `NWSRG Section 4 Salt Storage`.

    - Recommendations on winter resilience for English Local Highway Authorities were provided by The Quarmby Report of 2010, with the recommendation that Authorities should have sufficient pre-season stocks of salt for 12 days/48 runs (assuming each run at 20g/m²)

    - Initial Minimum Resilience for the full winter period has accordingly been calculated based upon heavy snow for a period of 12 days with assumed treatments of 20 grammes per square metre on each of the four priority routes with 4 successive treatments on each day.

The following table 1 for minimum salt stock levels is based upon the advice in NWSRG Section 4 Salt Storage

Table 1

| Minimum salt stocks in depot |  |  |  |
| --- | --- | --- | --- |
| Route | Normal Salting Network (Tonnes/run) | Minimum Winter Network Tonnes/run | Minimum Stock |
|  |  |  | Full Pre-Season Stock (12 days/48 runs) |
| Priority 1 Carriageways | 40.6 | 20.3 | 1948 |
| Footways cycle routes and salt bins | 4 | 4 | 192 |
| Total |  |  | 2140 |

    - The Minimum salt stock for both the core and overall winter period is

therefore 2140 tonnes (to the nearest 5 tonnes)

  - Legislative background.

    - In England and Wales Section 41 (1A) of the Highways Act 1980 was modified on 31st October 2003, by Section 111 of the Railways and Transport Act 2003. The first part of Section 41 now reads:

a)                 The Authority, who is for the time being the Highway Authority for a highway maintainable at the public expense is under a duty, subject to subsections (2) and (3) below, to maintain the highway.

b) (1)            In particular, a highway authority is under a duty to ensure, so far as is reasonably practicable, that safe passage along a highway is not endangered by snow or ice.”

    - This is not an absolute duty, given the qualification of “reasonably practicable” However Section 150 of the Highways Act 1980 still imposes a duty upon authorities to remove any obstruction of the highway resulting from “accumulation of snow or from the falling down of banks on the side of the highway, or from any other cause”.

    - In addition, the Traffic Management Act 2004 placed a network management duty on all local traffic Authorities in England. It requires authorities to do all that is reasonably practicable to manage the network effectively to keep traffic moving. In meeting the duty, Authorities should establish contingency plans for dealing promptly and effectively with unplanned events, such as unforeseen weather conditions, as far as is reasonably practicable.

    - Given the scale of financial and other resources involved in delivering the Winter Service it is not reasonable either to:

- Provide the service on all parts of the Network

- Ensure running surfaces are always kept free of ice or snow, even on the treated parts of the network.

- Route Planning for Carriageways, Footways and Cycle Routes

  - Carriageway routes by risk level

    - There are six different categories of Carriageway gritting response: -

- Priority 1 are major roads and bus routes (pre-treatment). There are four priority 1 routes that generally comprise all classified roads and bus routes. These routes which includes the full resilient network, cover 160km of road network, which is 27% of the total network length.

- Priority 2 are town Centre parking areas and some minor roads (pre-treatment nightshift only) There are two priority two routes that generally comprise town centre car parks

- Priority 3 are estate feeder roads (post treatment dayshift only). These are the internal estate feeder and through roads comprising 7 different routes

- Spot gritting-based upon thermal map information and scouting on marginal nights

- Emergency routes-for use only if the weather changes without warning and there is not sufficient time to treat the main routes

- Minimum routes—only to be utilised during times of severe salt shortages—there are two pre-defined minimum routes prepared for externally imposed salt saving requirements. These routes detail a 25% and a 50% reduction in the primary routes

    - Details of road salting routes are detailed in appendix B

    - The routes are reviewed regularly and are altered when the network changes i.e., Lindisfarne and The Arches.

2.2.   Response and Treatment Times for all Carriageway Treatments

    - Due to the maintenance of a permanent nightshift operation the response time from issue of instruction to the gritters leaving the depot to commence gritting operations will be within 1 Hour.

    - The time taken to complete any gritting route using a standard 20 grammes application rates should not exceed 3 hours.

  - Routes for Footbridges, Subways, and other High-Risk Pedestrian Areas

2.3.1. 	There are 7 different categories of footway gritting route that we would prioritise in the following order: -

- Steps, Footbridges, Ramps

- Shopping Areas

- Steep Footpaths, Outlying Areas

- Busy Footpaths (identified by highways)

- School Locations

- Public Buildings

- Residential Care Homes

  - Response and Treatment Times for Footway and Cycle Route Treatments

    - The response time for the treatment of all footpath and cycle routes is dependent on the resources available to the Streetscene manager and the severity of the weather conditions. We would always react to forecasts of inclement weather and prioritise resources to enable as many residents as possible to go about their daily lives in safety.

  - Routes for Other Footway and Cycle Route Treatment by Risk Level

- Steps, Footbridges, Ramps

- Shopping Areas

- Steep Footpaths, Outlying Areas

- Busy Footpaths (identified by highways)

- School Locations

- Public Buildings

- Residential Care Homes

2.6    Allocation of Plant, Vehicles, Equipment, and Materials to Routes

    - The Authority lease a fleet of five purpose built, 6 tonne capacity gritting wagons with detachable ploughs which allows the four priority 1 Carriageway Salting routes to be undertaken simultaneously with the extra vehicle providing resilience in the case of breakdowns.

    - When the priority 1 routes are completed the same vehicles may then commence the priority 2 routes if required.

    - Priority 3 routes are only salted by dayshift during prolonged weather events when all main roads are clear.

  - Location and Maintenance of Salt Bins and Grit Bags

    - Salt storage bins are located on the highway across the Borough at potential problem locations. They are generally not sited on the primary salting routes and are provided for public use on the public highway and to support footway-salting operations.  A list of these is shown in Appendix C and displayed on the Councils website.

    - All salt bins are inspected prior to the winter season by the highways staff. Litter and contaminated salt are removed, and all bins refilled.

    - Salt bins will be restocked during the season at the discretion of the duty manager as resources allow.

    - The Council will consider requests for the provision of salt bins where:

- The highway is on a steep incline (with a gradient over in 25)

- The highway connects to major roads or other junctions on a

downward incline leading to the major road

- The highway contains severe bends

- The highway carries high levels of traffic

    - The decision on whether to provide a grit bin will be based on consideration of the location meeting 2 or more of these factors and where there is a convenient area to site the grit bin and, in the case of residential areas, where the majority of residents are in favour of the proposals to provide a grit bin.

- A salt bin shall not obstruct the passage of pedestrians

- Salt bins for public use on the public highway will not be provided on a priority road or footway-salting route,

- A salt bin will not be provided within 50 metres of another salt bin.

    - Salt bins may be provided for other Council Departments and at locations that do not meet the above criteria on receipt of an official order. Such bins will not be topped up without an official order. Salt bins can also be supplied to private businesses and restocked for a fee depending upon salt availability and resources. There are currently 216 recorded salt bins on the adopted highway.

    - Restocking of some or all salt bins may be suspended during times of prolonged bad weather should salt supplies run low or the salt be taken for use on private property to the detriment of highway users.

    - In times of extreme and prolonged bad weather temporary grit bags may be placed at strategic locations to assist the public. This service will be at the discretion of the duty winter manager dependent upon available resources.

    - Salt bin locations are subject to periodic review and historic bins that do not meet the criteria may be removed.

  - Special Sites or Features (e.g., near railways or traffic calming)

There are three railway level crossings on the priority 1 road gritting routes: -

- Tileshed lane

- Benton Road

- East Boldon

    - Guidance from Network Rail requests that salt applied as grit is not laid any closer than 12 metres from the running rails on either side of any level crossing.

    - The benefits to the rail network are stated as: -

- Not applying grit from a distance of 12 meters from the nearest running rail both sides of the track will reduce (although not eliminate) the extent of corrosion and delays associated with track circuit failures

- The grit carried over by the wheels of motor vehicles applies deposits with comparable effects on skid resistance and sufficient to prevent the icing over of level crossing surface systems without the saturation of the crossing surface system and track bed

    - It is then up to the local network rail operations risk control coordinator to consider the provision of salt bins on a case-by-case basis.

##### Weather Prediction and Information

  - The Decision-Making Process

3.1.1. 	Decision making is the responsibility of the appropriate winter service manager for the road network and the Streetscene manager for the footway network.

3.1.2.	The winter service manager also has responsibility for direct liaison with the forecast provider and bureau service.

    - The press office will deal with all press enquiries with the duty manager and Streetscene managers providing updates as required to the press office for road and footpath issues respectively

    - Using the forecast data provided via the bureau service the winter service manager will make timely decisions regarding pre salting as appropriate to the road network

    - Using the same forecast data and working together with the winter service manager the Streetscene manager will evaluate the current situation and decide if pre-salting of footway areas would be beneficial.

    - Appropriate training is provided regularly for all winter service managers, particularly with regard to improvements in forecasting techniques and the ice prediction system

  - Road Weather Information Bureau Service

3.2.1.	The Authority utilise a regional framework currently awarded    to `Vaisala` for the provision of a road weather information bureau service.

3.2.2.	Information from all the weather stations across Tyne and Wear is available to the partners to better inform duty winter managers of prevailing or approaching weather conditions.

  - Road Weather Stations

    - The Authority has three weather stations: -

- Sunderland Road (Primary station)

- Hedworth Lane/Abingdon Way (shared with Autolink for A19)

- Mill Lane (Camera Only)

    - The station locations are displayed on the map in appendix D and displayed on the council’s website.

  - Timing and Circulation of Information

    - Winter service managers can access current weather forecasts from any location by logging on to the Vaisala website using their individual passwords.

    - This information is available on request but is not generally circulated beyond the direct decision makers

  - Road Weather Forecast

    - The Authority utilise information from a weather forecast provider. For the season 2022-23 this will be DTN EUROPE UK LIMITD Europe UK Limited (previously Meteogroup but company merged with DTN EUROPE UK LIMITD and renamed July 2022) who have won a regional NEPO framework tender to supply the service.

    - The forecasts from the service provider are accessed via the Vaisala bureau website or via DTN EUROPE UK LIMITD Europe UK Limited  own software and take the following form.

- About 1pm - Detailed 36hr forecast with anticipated freezing times. Road surface temperature graph with predicted freezing times as well as short term forecast and a 2–5-day forecast used to plan resource requirements (with particular relevance for establishing requirements for weekend operations)

- About 6pm a fresh detailed forecast similar to the 1pm forecast

- Hourly updates

    - Urgent amendments to the forecast may be received at any time from the forecast provider with the first point of contact being the Councils contact centre, which is in operation 24/7 for 365 days. They will pass the information on to the nightshift senior operative or winter service manager depending upon the time of receipt.

    - An example of the information received from the provider is detailed in appendix E.

  - Reporting Procedure

    - There is no formal procedure for disseminating the forecasts beyond the winter service duty managers. Should weather conditions be severe, senior officers will be kept up to date as required.

    - During extreme, severe or snow conditions information will be provided to the press office for circulation to local press and radio outlets with updates being placed on the Councils website and members being updated as appropriate to the circumstances.

  - Thermal Mapping

3.7.1.	On a typical winter night, the differences in temperature across a road network can vary by as much as 10ºC/18ºF. Consequently, some sections may be below freezing while others may not.

    - Thermal Mapping is a process by which the variation of minimum night-time road surface temperature is measured, using a high-resolution infrared thermometer.  It is a technique, which has been utilised worldwide, to enhance the information available to both highway authorities and supporting forecast providers.  Thermal Mapping is an integral part of an effective Ice Prediction system as it provides a mechanism for extending point specific sensor site information between individual weather stations and across a road network.

    - Vaisala remapped South Tyneside in 2020. The facility exists within the Vaisala website to time shift the thermal map through the night to identify potential problem areas at hourly intervals. An example of a thermal map display is shown in appendix F.

  - Maintenance of Ice Detection Equipment

    - As part of their current contract Vaisala maintains weather stations and ice detection equipment

    - Routine maintenance and upgrade works are carried out during the summer.

  - Information to be Provided.

    - Vaisala notify the Authority of any maintenance required to ensure the three weather stations remain operational.

- Organisational Arrangements and Personnel

4.1.	Command, Control, and Operational Organisation

Corporate Direct, Regeneration and Environment

Roads
Infrastructure and Transport Service Area – Highways and Infrastructure Manager

Footways
Streetscene Managers

Dayshift Supervisor

Weekend Drivers

Dayshift Drivers

Nightshift Drivers

Winter Service Duty
Managers

Streetscene Operatives

  - Arrangements with Other Authorities

    - The following co-operative arrangements have been made with adjacent authorities.

    - Sunderland City

- Whitburn Boundary - Treat to Boundary only.

- Follingsby Lane	Continue to Junction with A194.

- Hylton Lane Continue to Downhill Bus Terminus and Return

(Double).

4.2.3.           Sunderland Council

- Hebburn Boundary.

- Treat to Boundary only.

  - Arrangements with other public services

    - The Authority has arrangements to salt

- South Tyneside District Hospital main internal ring road

- Mill Dam Police Station

- South Shields Transport Interchange

  - Decision Making

4.4.1. 	          Council decision making process

Morning – Check last nights actions and update as required.

Close any completed operations

1:00pm – Check main forecast including graph of road surface temperatures and projected thermal map – make decision and enter onto system or defer if required due to forecast uncertainty

4:00pm – Call in night shift early for 7pm (if required)

10:00pm (or 7:00pm) – Call nightshift supervisor and confirm actions.

4.4.2. 	          External fri/sat nightshift contractor process

Every Wednesday 1:00pm – Check forecast and email contractor outlining initial view of resource requirements for Friday / Saturday

Every Thursday 1:00pm – Check forecast and email contractor with final confirmation of resource requirements for Friday / Saturday

4.5. 	Operational record keeping and reporting

    - Daily operational decisions made by the winter service manager for road treatments are recorded daily on the Vaisala Manager software (Appendix G) as actions to the drivers and in the online diary.

    - The Manager system is updated by the nightshift senior operative at the end of his shift with regards to drivers/treatment start finish times/tonnages etc.

  - Plant and Vehicle Manning Arrangements, including Management of Driver’s Hour’s Regulations

    - During the winter period driver arrangements are as follows are as follows

- Sun/ Mon /Tues/ wed /Thurs -an on station night-shift comprising a minimum of four internal council HGV drivers including at least one supervisor

- Fri Sat- an on/station council night shift supervisor and a minimum of four `Aimrange` HGV drivers on call.

- All day-shift salting requirements 7 days a week  will be carried out by `Aimrange` on behalf of the council

    - All drivers, both council and Aimrange, are qualified to drive the loading shovel.

  - Driver’s Hours

    - Driver’s hours comply with the Domestic driving rules

- Maximum Daily Driving of 10 Hours.

- Maximum Daily Duty of 11 Hours.

- Minimum of 30 minutes break taken rising to 45 minutes if shift extended.

- Drivers’ hrs are managed via a combination of manual and digital tachograph records

    - Fortnightly rest periods - In any 2 weeks in a row (Monday to Sunday) there must be at least 1 period of 45 hours off.

#### Materials Management

    - The salt stocks are monitored on a daily usage basis. The tonnage of salt used on each salting route is known and the daily usage is recorded by the duty manager through the Vaisala software system.

    - The Council will normally commence the winter season with a salt stock of 7000 tonnes.

  - Training and Development Arrangements

    - All winter service managers have been trained in the interpretation of weather forecast information and by Vaisala in scenario training. Regular refresher retaining is undertaken.

    - All drivers of the Salt Spreading Vehicles receive proper training and instruction in the safe and efficient operation of the vehicles and their equipment.

  - Schedules of Contract and Voluntary Personnel (CVP)

    - HGV driving operations are carried out by a mixture of internal council HGV drivers and external drivers as detailed in 4.6.1

    - All footway salting operations are carried out by internal council operatives.

    - In times of extreme snow events Community payback teams and/or local contractors may be utilised or contracted on an ad hoc basis to provide extra support in delivering the service.

    - Community groups who indicate a desire to assist in clearing snow from footways may also be supported

  - Employee Roles and Responsibilities

    - Highway officers undertake the role of managing the delivery of the primary road gritting service

    - Streetscene managers undertake the role of managing the delivery of the footway gritting and footway snow clearing service

  - Contact and Commissioning Arrangements for CVP

    - The Authority has no formal arrangements for utilising voluntary personnel. It is however recognised that members of the public or community groups may clear or grit areas of public highway utilising resources from grit bins on the public highway. Central Government have published the `Snow Code` as a guide to the issues that arise from the public clearing snow from the public highway (Appendix H).

  - Employee Duty Schedules, Rotas, and Standby Arrangements

    - There are five trained winter service managers within the Infrastructure and Transport team. A weekly rota of winter duty managers is operated throughout the winter period. Each winter service manager will be responsible for all road gritting duties carried out during his allotted duty period in accordance with this document.

    - The streescene managers are jointly responsible for all footway gritting operations throughout the winter period in accordance with this document. Weekend services will only be provided in times of extreme and prolonged bad weather.

  - Winter Service Exercising Arrangements

    - No pre-season exercises are undertaken.

  - Standard Operating Procedures

    - Duty manager checks forecasts, issues, and records instructions regarding proposed treatments of roads.

    - Streetscene managers check forecasts and issue instructions regarding footway treatments.

  - Escalation and Emergency Operating Procedures

    - Should the weather conditions deteriorate suddenly or vary significantly from that forecast and planned for, the nightshift senior operative is authorised to increase spread rates and/or fit/commence ploughing without first contacting the duty winter manager. He should however confirm these actions to the winter service manager by e mail at the first available opportunity and record his actions on the appropriate software diary.

    - If a period of prolonged severe snowfall should occur, then one member of the winter service manager’s team will be seconded from normal duties and teamed with a Streetscene manager/operative to provide a combined and coordinated response for the duration of the weather event.

    - Should weather conditions deteriorate without warning overnight the emergency road gritting routes will be treated.

    - Should the Councils stockpile of salt fall below a resilience of 12 days then the winter service manager may, in consultation with the Head of Infrastructure and Transport, begin to salt a reduced network of roads and footways. (Any further reduction would be undertaken under guidance from the Dept of Transport Resilience team or the Secretary of State).

  - Operational Monitoring

    - The effectiveness of the daily operations will be monitored by the relevant winter service manager and Streetscene manager for their own areas of responsibility.

  - Health and Safety Procedures

    - Nightshift crews operate from a mess facility within the Middlefield’s depot with appropriate facilities for health and welfare.

    - Coshh assessments for salt are provided to all operatives and are detailed in appendix I.

    - Risk assessments for the following operations are detailed in appendix J.

    - Gritting

    - Loading of gritting vehicles

  - Contingency Arrangements.

    - Breakdown of gritting vehicles/Damage to Ploughs—The Authority have a contract with ECON for maintenance of the 5 hired gritting vehicles on a 24-hour basis

    - The two spare council owned gritters are only maintained during normal working hours.

    - Breakdown of primary loading vehicle—The Council highways team have access to a 3cx excavator with front loading 4 in 1 bucket that can be utilised should the primary loading vehicle be unavailable.

    - The five Tyne and Wear authorities have a mutual aid agreement should there be a shortage of salt. Salt may also be purchased from the National Strategic reserve if other supply options are exhausted.

    - Mutual aid arrangements are also in place for gritting vehicles. Alternatively, gritting vehicles would be hired in from appropriate suppliers.

    - 24-hour contact numbers are available for duty managers to contact both the appropriate weather forecast provider and the Vaisala weather bureau should the computer systems fail for any reason.

    - Should all electronic forms of communication fail the winter service manager will personally attend the depot to direct operations.

- Facilities, Plant, Vehicles and Equipment

5.1.    Winter Service Compounds and Facilities

5.1.1.	The main compound for the winter service is within the Middlefield’s depot. This contains: -

- The winter cabin comprising messing facilities for HGV drivers

- Parking for the five hired and two internal gritting vehicles

- The 3cx (primary loading vehicle) utilised for loading the Gritting wagons

- The backup loading shovel

- The stockpile of salt

- Vehicle maintenance services

- The fuel stores.

5.2.        Calibration Procedures

5.2.1.	Econ recalibrates the spinners on the gritters just prior to each new winter season.

5.2.2.	The calibration of the salt spreading equipment shall be in accordance with BS 1622. Calibration test record sheets will be provided to fleet manager prior to the start of the WM season.

5.3.	Fleet inventory Including Licence Requirements and Capacity

5.3.1.	 Five hired gritting vehicles-6 Tonne Capacity and two council owned gritters

5.3.3.	       20 hand salters for hard-to-reach footway areas.

5.4.	Fuel Stocks and Locations

    - Middlefield’s depot houses its own internal fuel stock.

  - Location of Plant, Vehicles, Snow Blowers and Other Equipment

    - All equipment is stored at Middlefield’s depot.

  - Contingency arrangements

    - The Middlefield’s depot has a second entrance that can be used in times of emergency.

    - Should the whole depot and gritting fleet be rendered inaccessible the Authority would source alternative gritting vehicles from external hire companies if available.  Failing that a request for mutual aid would be made of neighbouring Tyne and Wear Authorities for gritting vehicles and or salt.

    - The issue of daily instructions by the winter service managers can be undertaken from any location with Internet access and is not dependent upon access to the Middlefield’s depot.

  - Garaging, servicing, and maintenance arrangements

    - The five hired Econ gritting vehicles are maintained under a maintenance agreement directly by Econ.

    - The two council owned spare gritters are provided in house by the Authorities fleet management team based in Middlefield’s depot except for: -

- The 3cx that is hired from and maintained by George Vardy Ltd.

- The backup loading shovel that is on site hired from and maintained by George Vardy Ltd.

  - Contact and Hire Arrangements for Contract Plant.

    - Council fleet manager—all internal council vehicles.

    - George Vardy---loading shovels

    - Econ-5 Hired Gritters

- Salt and Other De-Icing Materials

  - Location and Capacity of Stocks for Salt and Other Materials

    - The main salt stockpile is within the Middlefield’s depot. The stockpile at the start of the winter season will normally contain 7000 tonnes.

  - Contacts and Purchasing Arrangements for Supplies

    - Salt is supplied under tender via a NEPO contract with ICL operating from the Boulby mine at Saltburn, Cleveland.

  - Minimum Pre-Season and In-Season Stock Levels

    - The salt stockpile is normally restocked in during the summer to take advantage of seasonal variations in price.

    - The minimum in season stock level is 2140 tonnes, which is equivalent to the minimum 12-day resilience level in accordance with national guidance.

  - In season Re-Stocking Arrangements

    - For in season restocking contact ICL to confirm details by telephone and place order through internal ordering systems.

#### Testing Coarse Rock Salt’ (10mm particle size) Arrangements

    - Samples of salt will be tested as required. The salt that is used on highways during winter is a naturally occurring rock salt, and shall be to the requirements of BS 3247 ‘Salt for Spreading –

  - Stock level Monitoring and Forecasting Procedures

    - Salt usage is recorded throughout the season daily on the relevant software that also displays the quantity of salt remaining for resilience monitoring.

  - Loading Arrangements

    - Salt is loaded directly into the gritting vehicles by the primary loading vehicle that is available within the depot.

  - Treatment Requirements Including Spread Rates.

    - Treatments as a minimum meet and in some instances exceed national updated guidance in `NWSRG section 8-Spread rates for preactionary salting`.

  - Road Surface Wetness

    - For allocating treatments, a distinction is made between dry, damp, and wet road surfaces. The following definitions drawn from `NWSRG-section 8-guide to spread rates` and should be used when making the treatment decision.

Table 2. Road surface wetness

| Table 2 – Road Surface Wetness |  |  |
| --- | --- | --- |
| Definition | Description | Water film thickness (for when using WFT instrumentation) |
| Dry road | A road that shows no signs of water or dampness at the surface but may be just detectably darker. It may have moisture contained in pores below the surface that is not ‘pumped’ to the surface by traffic. | 0 to 0.03mm (=0-30 g/m2) |
| Damp road | A road which is clearly dark, but traffic does not generate any spray. This would be typical of a well-drained road when there has been no rainfall after 6 hours before the treatment time. | 0.03 to 0.05mm (=30-50 g/m2) |
| Wet road | A road on which traffic produces fine spray but not small water droplets. This would be typical of a well-drained road when there has been rainfall up to 3 hours before the treatment time. | 0.05 to 0.1mm (=50-100 g/m2) |
| Very Wet Road and Flowing Water on Road* | A road on which traffic produces droplets of water in the air to visibly flowing water on the surface. | Greater than 0.1mm (=>100 g/m2) |

  - Traffic Levels

    - For allocating treatments, the guidance defines two levels of trafficking on roads – heavily trafficked roads and medium/light trafficked roads.

    - As shown in table 3 below, heavily trafficked roads are those defined as Categories 1 and 2 in Well Maintained Highways and medium/light trafficked roads are those defined as Category 3.

Table 3-Carriageway Hierarchy

| Table 3 – Carriageway Hierarchy |  |  |
| --- | --- | --- |
| Category | Hierarchy Description | Traffic Level |
| 1 | Motorway | Heavy |
| 2 | Strategic Route | Heavy |
| 3a | Main Distributor | Medium/Light |
| 3b | Secondary Distributor | Medium/Light |

  - Precautionary treatment matrix

    - A Decision Matrix Guide is utilised rather that a flow chart as it contains more detail for the decision maker

    - A decision matrix for precautionary treatments based on road surface conditions and predicted weather conditions is given in Table 4.

Table 4-precautionary treatment decision matrix

| Table 4 – Precautionary Treatment Decision Matrix |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Predicted Road Conditions |  |  |  |
| Road Surface Temperature | Precipitation | Wet/Damp | Wet Patches |  | Dry |
| May fall below 1 oC | No rain No hoar frost No fog | Salt before frost | Salt before frost (see note a) |  | No action likely, monitor weather (see note a) |
| Expected to fall below 1 oC | No rain No hoar frost No fog |  |  |  |  |
|  | Expected hoar frost Expected fog |  |  |  |  |
|  |  |  | Salt before frost (see note b) |  |  |
|  | Expected rain BEFORE freezing | Salt after rain stops (see note c) |  |  |  |
|  | Expected rain DURING freezing | Salt before frost, as required during rain and again after rain stops (see note d) |  |  |  |
|  | Possible rain Possible hoar frost Possible fog | Salt before frost |  | Monitor weather conditions |  |
| Expected Snow (see Section 6.14) |  | Salt before snow fall |  |  |  |
| The decision to undertake precautionary treatments should, if appropriate, be adjusted to take account of residual salt or surface moisture. All decisions should be evidence based, recorded, and require continuous monitoring and review. |  |  |  |  |  |

# Notes:

- Particular attention should be given to the possibility of water running across carriageways and other running surfaces e.g., off adjacent fields after heavy rains, washing off salt previously deposited. Such locations should be closely monitored and may require treating in the evening and morning and possible other occasions.

- When a weather warning contains reference to expected hoar frost, considerable deposits of frost are likely to occur. Hoar frost usually occurs in the early morning and is difficult to cater for because of the probability that any salt deposited on a dry road too soon before its onset, may be dispersed before it can become effective. Close monitoring is required under this forecast condition which should ideally be treated just as the hoar frost is forming. Such action is usually not practicable, and salt may have to be deposited on a dry road prior to and as close as possible to the expected time of the condition. Hoar frost may be forecast at other times in which case the timing of salting operations should be adjusted accordingly.

- If, under these conditions, rain has not ceased by early morning, crews should be called out and action initiated as rain ceases.

- Under these circumstances rain will freeze on contact with running surfaces and full precautionary treatment should be provided even on dry roads. This is a most serious condition and should be monitored closely and continuously throughout the danger period.

- Weather warnings are often qualified by altitudes in which case differing action may be required from each depot.

- Where there is any hint of moisture being present, a pessimistic view of the forecast should be taken when considering treatment to negatively textured surfaces.

  - Determining Spreading Capability

    - For precautionary treatments, the spread rates to be used depend upon the spreading capability. Capability in national guidance is defined as either “good” or “fair”. Due to the Authorities salt being stored in the open the moisture content of the salt is variable which results in a “fair” capability for spreading.

    - For a fair level of spreading capability, the spread rates used should be those given in Table 4. These rates are marginally higher than the minimum recommendations given in NWSRG series 8 (This standard is adopted by South Tyneside due to the salt being stored outside which makes moisture content inconsistent.)

  - Spread Rates for Precautionary Treatments (forecast frost conditions)

    - Spread rates for precautionary treatments before frost are given in Table 4. The table provides recommended spread rates for dry salting for a range of weather and road surface conditions.

# NOTE: 	The following points must be considered when using the spread rate tables.

- The given spread rates are for sections of well drained roads without ponding or runoff from adjacent areas.

- The rates may be adjusted to take account of variations occurring along routes such as temperature, surface moisture, road alignment and traffic density.

- The rates may be adjusted to take account of residual salt levels. However, residual salt levels will tend to be lower if lower spread rates are introduced. Residual salt levels are most likely to be significant on marginal nights after treatments on two or three successive days without precipitation in the intervening period.

- On porous asphalt and on dense surfacing for 1km after a change from porous asphalt, spread rates should be increased by 50 per cent on roads with medium traffic levels and by 25 per cent on heavily trafficked roads.

- Spread rates should be increased to a rate appropriate for the situation where negatively textured thin surfacing’s are poorly drained such that water can accumulate within the surface texture.

- The performance of spreaders should be monitored, and checks should be made that the amount of salt discharged is within 10 per cent of the target and that treatments are effective.

- All decisions should be evidence based, recorded, and require appropriate monitoring and review.

- During periods of sustained freezing and provided that surfaces are well drained and there is neither seepage (from melt water) nor ice present, rates of spread for treatments carried out within six hours of previous treatments may be 50% of the rates stated in the appropriate table.

Table 5- recommended spread rates for dry salting

| RECOMMENDED SPREAD RATES DRY SALTING G/M2 FROM NWSRG MATRIX 8.6.7 (in grey) AMMENDED UP 20% FOR unprotected SALT then rounded to nearest multiple of 5(in bold) |  |  |
| --- | --- | --- |
| Road surface temperature when frost/ice is predicted | Spreader capability |  |
|  | Fair |  |
|  | Dry/damp Rd | Wet Road |
| At or above -1.0°C | (10) | (10) |
| -1.1°C to -2.0°C | (10) | (15) |
| -2.1°C to -3.0°C | (15) | (25) |
| -3.1°C to -4.0°C | (15) | (30) |
| -4.1°C to -5.0°C | (20) | (35) |
| -5.1°C to -7.0°C | (25) | (2x25) |
| -7.1°C to -10.0°C | (35) | 2x(35) |
| -10.1°C to -15.0°C | (2x25) | (2X40 successive) |
| Rate of spread for precautionary treatments may be adjusted to take account of residual salt or surface moisture as per the notes |  |  |

  - Treatments for Snow and Ice

    - Whenever practicable, it is crucial that de-icer is spread on road surfaces before snowfall and freezing rain occurs. This provides a layer which prevents the snow and ice bonding to the road surface (debonding layer), and this aids subsequent treatments.

    - Based around three types of snow: dry (powdery), normal and wet National guidance uses two categories of snow intensity: light snow and moderate/heavy snow for the purposes of allocating treatments.

    - The highest spread rate used by most authorities for dry salting is 40g/m2 (although it is recognised that delivering this onto the road surface may require more than one treatment). This spread rate, combined with the effect of trafficking, should be sufficient to melt and disperse snow depths which are equivalent to a maximum of 1mm of water at temperatures down to -2°C but will not be sufficient to melt heavier snow.

  - Timing of treatments

    - When snow is forecast and issues of practicality allow, ploughs and snow blowers should be made ready to allow snow clearance to commence without delay as and when required. The recommended aim is to apply a precautionary treatment to the salting network immediately prior to snow fall or freezing rain, as this should significantly reduce the risks of snow settling or ice forming on the road surface.

    - When feasible, treatments should be carried out after any preceding rainfall has ceased and sufficient time and traffic has removed excess water on the road surface. It is, however, recognised that there may be insufficient time during the intervening period to undertake a full precautionary salting operation. These situations are some of the most difficult of all to deal with and, whenever snowfall or freezing rain is expected, decision makers will need to carefully consider the most appropriate and effective timing for operations. In some circumstances, treatments may be required to be undertaken during rainfall or on very wet road surfaces and repeat treatments may be required to prevent compacted snow or icy conditions occurring as snow continues to fall.

    - When snow is settling, ploughing should be carried out at an early enough point to prevent its compaction by traffic. During prolonged snowfall events and when issues of practicality allow, the time periods between successive ploughing passes necessary to prevent a build-up of snow should be kept short enough so that compaction is minimised. In some instances, this may require continuous ploughing operations.

    - Depending on the prevailing conditions, subsequent treatments should be carried out as shown in table 6 below:

Table 6- weather condition and treatment types

| Timing of treatment During freezing rain, or where there are minor accumulations of ice | Treatment type Salt spreading |
| --- | --- |
| During snowfall | Ploughing and salt spreading |
| After snowfall when there is slush on the road When there is compacted layers of snow and ice | Ploughing and salt spreading Salt and abrasive mixtures or just abrasives can be used on compacted layers of snow and ice |

  - Freezing Rain

    - Freezing rain is a relatively rare, but hazardous phenomenon in the UK, as the nature of freezing rain means that the risk of ice formation is high, even on treated surfaces. When the rain strikes a surface, it freezes to form glaze ice almost immediately on contact.

    - When freezing rain is forecast, precautionary salting should be applied, however it should be recognised but the risk of ice formation on the roads remains high in these situations

  - Snow ploughing

    - The purpose of ploughing is to move as much snow as possible away from the road surface. More than a few millimetres of snow cannot be treated with salt.

    - Snowploughs should be fitted to vehicles 24 hours before snow is forecast

  - Treatments Before Snowfall and Freezing Rain

    - It can be very difficult to remove a layer of compacted snow or ice that is bonded to a road surface and a debonding layer is important so that:

- Snow is more readily removed by ploughing

- Compacted snow and ice are more easily dispersed by traffic

- To provide salt to melt small quantities of light snow with the aid of trafficking

Table 7- treatments before snowfall and freezing rain

| # Treatments Before Snowfall and Freezing Rain |  |
| --- | --- |
| Weather conditions | Spread: |
| Light to Moderate/Heavy snow forecast | 20-40g/m² of dry salt, or l 20-40g/m² of pre-wetted salt, or l 15-30g/m² of treated salt |
| Freezing rain forecast | 40 or 2x20g/m² of dry salt, or l 40 or 2x20g/m² of pre-wetted salt, or l 30 or 2x15g/m² of treated salt |
| In situations where time constraints dictate, a treatment of 20g/m² across the whole of the scheduled network before the commencement of snowfall or freezing rain will typically prove more advantageous than a treatment of 40g/m² on only part of the network. |  |

  - Treatment During Snowfall and Freezing Rain

    - Treatments are undertaken during snowfall to:

- Limit the accumulation of snow on the road surface, thereby reducing the amount of salt required for subsequent treatments

- Help the dispersal/clearing of the snow by traffic

- Prevent snow from being compacted

# Table 8 treatment during snowfall and freezing rain

| # Treatment During Snowfall and Freezing Rain |  |  |
| --- | --- | --- |
| Plough to remove as much material as possible e.g., slush, snow, compacted snow Ploughing should be down to the level of the road surface Ploughing should start and, where necessary, be continuous to prevent a build-up of snow As snow melts under the action of salt, keep ploughing to remove slush |  |  |
| No ice or compacted snow on surface | Ice or compacted snow on surface |  |
| To provide a debonding layer, spread: L 20-40g/m² of dry salt, or L 15-30g/m² of treated salt or l 20-40g/m2 of pre-wetted salt | Is traffic likely to compact subsequent snowfall before further ploughing is possible? |  |
|  | YES | NO |
|  | To provide a debonding layer, spread: l 20-40g/m² of dry salt, or l 15-30g/m² of treated salt or l 20-40g/m2 of pre-wetted salt | No de-icer should be spread |

    - De-icer should not be spread alone without abrasives to anything other than a thin layer of ice or compacted snow when snowfall has ceased, or future snowfall will be less than 10mm. Applying salt alone to compacted snow and ice can produce dangerously slippery conditions if a weak brine film is formed on top of the ice/snow layer.

  - Treatments of Thin Ice Layers

    - Treatments using salt or salt and abrasive mixes should be made in accordance with Table 9 below

# Table 9-treatments for thin ice layers

| # Treatments of Thin Ice Layers (Less than about 1mm thick) |  |
| --- | --- |
| Forecast weather and road surface conditions |  |
| Lower of air or road surface temperature # Above -5ºC | Spread: l 40g/m² of dry salt, treated salt or pre-wetted salt, or # l 40g/m2 of salt/abrasive mix |
| Lower of air or road surface temperature # At or below -5ºC | Spread: # l 40g/m2 of salt/abrasive mix (50:50) |
| Note 1: Salt is ineffective in the short term at temperatures below -7°C. Abrasives only should be used when it is expected to be below -7°C for long periods. Other de-icers are available for low temperatures (see De-icer Types Section). |  |

  - Treatment of Compacted Layers of Snow and Ice

    - Treatments should be made in accordance with Matrix 9.11.3

    - For compacted snow, when no further snow is expected, salt and abrasive mixtures or abrasives are applied to assist the action of traffic in breaking the layer. When further snow is expected, salt and abrasive mixtures treatments may be applied to provide grip as well as a debonding layer between the existing layer and any future snow to assist future ploughing operations.

    - Salt should not be applied on its own as it may eventually form a weak brine solution which may re-freeze to form an ice or ice/brine layer.

# Table 10-treatments for layers of compacted snow and ice

| Table 10 Treatment for Layers of Compacted Snow and Ice |  |
| --- | --- |
| Plough to remove as much material (e.g., slush, snow, compacted snow) as possible from the top of the compacted layer |  |
| Medium Layer Thickness (1 to 5 mm) | High Layer Thickness (greater than 5mm) |
| For initial treatment, spread: l 40g/m2 of salt/abrasive mix (50:50) F or successive treatments, spread: # l 20g/m2 of salt/abrasive mix (50:50) | For initial treatment, spread: l 40g/m2 of abrasives only F or successive treatments, spread: l 20g/m2 of abrasives only After traffic has started breaking up the layer, spread: # l 20g/m2 of salt/abrasive mix (50:50) so salt can penetrate the layer and reach the road surface |

###### Operational Communications

  - Technical Systems Information

    - It is important that good communications are maintained during the winter period to ensure that the response to poor conditions is effective and immediate.

    - At all times the gritting crews should be available by mobile phone. Contact between vehicle operators and the duty manager is to be maintained at all times during winter maintenance operations, so that salting progress can be monitored and to provide a rapid and efficient response in emergency situations.

    - During normal working hours the duty winter service manager will be available via the Council’s Contact Centre or by mobile phone if they are away from the office.

    - The duty winter service manager has responsibility for managing the service outside normal working hours and is available via mobile phone.

## During severe periods of weather (snow and widespread ice) problem areas may be notified by phone calls received from the Police and members of the public via the Contact Centre. They will act as a liaison to the duty manager and log these calls along with any other reports.

  - Reporting Arrangements and Protocols

    - A decision is made and recorded every day even if that decision is to take no gritting action.

    - Decisions by the winter service managers are recorded on the Vaisala Manager system.

    - Decisions by the Streetscene managers are stored as per their procedures

  - Inventory and Allocation, Including Back Up.

    - Should communications or IT systems fail the duty winter manager will attend the Middlefield’s Depot to take control of operations in person.

    - The manager has 24HR contact helpline numbers for both the weather forecast provider and bureau service if required.

###### Contingency Plan

  - Contingency Arrangements for Winter Service Delivery Such as Salt Supply, Drivers, Fuel Vehicles etc

    - Salt. Extra supplies are available through the contracted supplier or via mutual aid. As a last resort salt can be purchased from the National Strategic reserve.

    - Drivers. The Authority utilise drivers in house who have experience of driving gritting vehicles to be able to replace any drivers who suddenly become unavailable. Drivers may be available under mutual aid from neighbouring authorities or from external agencies if required.

    - For dayshift salting the councils external gritting support contractor, Aimrainge,  has reserve trained drivers that it can call on if required

    - Fuel. The Authority maintains a supply of fuel cards which can be used at local garages.

    - Vehicles. Vehicles can be secured on hire through the council’s fleet management team. Alternatively, vehicles can be provided by neighbouring authorities under mutual aid.

  - Arrangements for Implementing Minimum Winter Networks

    - The decision to implement minimum winter salting networks cover will be taken by the Head of Infrastructure and Transport in consultation with the Service Lead, highways Asset Management Highways, Infrastructure Manager and Duty winter service manager.

    - This decision will be circulated to Members and the press office for the widest circulation and posted on the Councils website.

    - Details will include the reduced network to be salted and the reason for the decision along with any relevant guidance from the Dept for Transport.

#### Mutual Aid e.g., Resources Available from Adjacent Authorities

    - Mutual aid from neighbouring authorities will be dependent upon the circumstances that prevail at the time the aid is requested.

    - Typically, requests may be made for salt, gritting vehicles, or drivers.

#### Liaison with Category 1 and Category 2 Responders (Reference Civil Contingencies Act 2004)

    - Liaison with Category 1 and 2 responders is undertaken via the Councils Emergency Planning and Response Coordinator.

###### Information and Publicity

  - Local Press and Broadcast Contact Information

    - All contact with local press and broadcasters will be via the Councils press office.

#### Public Information Leaflets

    - The Highways Agency produces a public information leaflet on driving safely in winter. This can be viewed on https://nationalhighways.co.uk/

  - Other Key Local and National Contact Information

ICL (Salt Supplier)

01287 640140

  - For Issues on the A19

Autolink

Number held on Vaisala software for winter managers

  - For Issues on the A184  Trunk Road Contact

North East Area 14

Number held on Vaisala software for winter managers

  - For Council Risk and Resilience Matters Contact Corporate Manager-Risk and Resilience.

  - Thermal Mapping

    - Information on thermal mapping is shown on the Vaisala website at www.vaisala.com.

  - Responsibilities and Guidance for Providing Information

    - It is the responsibility of the Head of Infrastructure and Transport to provide guidance and information on winter maintenance matters.

  - The Decision-Making Process

    - All press releases and contact with external media will be via the press office. All press statements to be approved by the Head of Infrastructure and Transport prior to release.

  - Road Weather Stations

    - Information on the location of the three road weather stations is on the Councils website and shown in appendix D.

  - Information to be Provided

    - Information from the weather stations or weather warnings may be provided on the Councils website during periods of extreme weather.

  - Road Weather Information Bureau Service

    - Vaisala are the Authorities contracted road weather information bureau service. Extensive detailed information can be found on their website, www.vaisala.com.

  - Timing and Circulation of Information

    - Press releases approved by the Head of Infrastructure and Transport, released through the press office as appropriate.

#### Road Weather Forecast

    - DTN Europe UK Limitd (formerly Meteogroup) is the Authorities contracted road weather forecast provider

    - The weather forecasts are divided into three climatic zones covering the Tyne and Wear region. These zones are coastal central and western and are shown in appendix I.

  - Information to be Provided

    - Information provided to the press will be specific to the nature of the prevailing weather.

    - Members will be updated by e mail of any significant snow events and the Councils ongoing response.

  - Notification Arrangements for Failure to Maintain the Published Network

    - Any failure to treat the published network will be circulated to local media via the press office.

    - Local emergency services and bus operators will also be notified.

    - Members will also be notified by e mail.

  - Reporting Procedure

    - Any failures to salt the published network will be reported to Cabinet as part of an annual revision of this plan.

  - Maintenance of Ice Detection Equipment.

    - Ice detection equipment is maintained by Vaisala.

10. Quality Management

  - Arrangements for Performance Monitoring, Audit and Updating

10.1.1.	This service plan will be updated as required after each winter season and approved.

  - Document Control Procedures

    - The Head of Infrastructure and Transport must approve any deviation from the laid down procedures within this plan.

10.3.	Distribution of Documents

10.3.1.	All winter service managers will receive a copy of the winter service plan.

10.4.	Information Recording and Analysis

10.4.1.	All winter duty managers’ decisions will be recorded on the appropriate standard documentation and/or software.

    - Minutes will be kept of all meetings regarding the delivery of the service throughout the season and stored in the appropriate folder on the Authorities servers.

  - Arrangements for Performance Monitoring, Audit and Updating.

    - Performance will be monitored throughout the season.

  - Procedure for Deviation from the Winter Service Plan.

    - The Head of Infrastructure and Transport must approve any deviation from the laid down procedures within this plan.

  - Service Review following Significant Events and at End of Season

    - An end of season review meeting will be held within one month of the close of the service each year.

    - The results of the annual review meeting will inform the revisions and updates to the service plan for the next season.

Appendix A

Winter Maintenance Managers

Instruction and Guidance Manual

SOUTH TYNESIDE COUNCIL

WINTER MAINTENANCE PLAN

# WINTER MAINTENANCE DUTY MANAGER (ROADS)

INSTRUCTION AND GUIDANCE MANUAL

## Operational Period

The provision of weather forecasting services commences on the 1st of October. A limited permanent gritter driver nightshift and activation of weekly duty managers commences on or about 17th October and the full gritter driver nightshift cover commences on 1st November. This permanent core nightshift cover runs until 31st March when, if weather permits, the gritter driver numbers are reduced and then operate until about 14th April when conditions should allow the service to be terminated for the season.

A duty period of one week will be undertaken by each Manager commencing at 7am on Monday and lasting continuously for seven days until 7am on the following Monday.

During the duty period the Winter Maintenance Manager will remain available by mobile or land telephone and will remain within a distance from Middlefield’s Depot that can enable his return within less than one hour.

## Operational Readiness

On each week of his duty the Winter Maintenance Manager will check that the following facilities are adequate to meet any expected weather conditions for his period of duty as follows:

- Adequate quantities of rock salt in accordance with the resilience requirements

- Adequate gritting vehicles and snowploughs

- Loading shovel availability

- Stand-by maintenance arrangements for vehicles

- Manpower for night shift drivers

- Manpower for additional dayshift drivers

On each day of his duty the Duty Winter Maintenance Manager will check the following:

- The availability of gritting vehicles

- The availability of a loading shovel

- The attendance of all drivers at the required time

## Weather Report Interpretation Duties

He will be responsible for the viewing and interpretation of the weather reports provided by the forecast provider and the forecast graph, thermal maps and site station information provided by the winter bureau service provider.

The winter maintenance manager will check during the early part of each day, the morning forecast for the coming day to establish the weather conditions for the day and whether any action is required at that time.  The manager will make a record of any decisions or actions on his daily record sheet.

During the early afternoon the winter maintenance manager will view the midday 10 day and 36 hr forecast and consider the actions that will be appropriate for the forthcoming night. The forecast graph will also be viewed along with the thermal map for the appropriate decision to be made. The manager will make a record of any decisions or actions on his daily record sheet and enter the appropriate instructions on the Manager software provided by the bureau service.

In the event of the absence of the duty manager during any part of his duty period, due to business reasons or short-term leave, he must arrange for one of the other winter maintenance managers to view the forecasts and establish any required action and record the actions taken. However, upon return to duty the winter maintenance manager must immediately view all the forecasts and ice detection information, review any existing instructions and record confirmation of his agreement with the actions already instigated. The absence of the duty winter maintenance manager during his duty week must not include any absences beyond 5pm on any weekday or any time during a weekend or bank holiday.

## Weather Reports, Action, Decisions

In determining the decision being made from the interpretation of the weather report the winter maintenance manager is guided by the following code of practice guidance:

## DECISION MAKING PROCEDURE

(Established based on the code of good practice but adjusted to take account of the availability of a night shift system)

| ROAD CONDITIONS | ROAD SURFACE TEMPERATURE | May fall below freezing | Expected to fall below freezing (See note E) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | PRECIPITATION ETC | No Rain | No Rain | Expected Hoar Frost | Expected Rain Before Freezing | Expected Rain During Freezing | Possible Rain |
|  |  | No Hoar Frost | No Hoar Frost | Expected Frost |  |  | Possible Hoar Frost |
|  |  | No Fog | No Fog |  |  |  | Possible Fog |
| Wet |  | 1 | 1 | 1 | 3 See Note C | 1 See Note D | 1 |
| Wet Patches |  | 2 See Note A | 2 See Note A | 2&4 See Note B | 3 See Note C | 1 See Note D | 5 See Note F |
| Dry |  | 6 See Note A | 6 See Note A | 4 See Note B | 3 See Note C | 1 See Note D | 5 See Note F |
| Pre-salted within previous 24 Hours with no rain since |  | 6 See Note A | 6 See Note A | 6 See Note A | 3 See Note C | 1 See Note D | 5 |

Action Table

Action 1	Pre-salt minimum of 3hrs before forecasted onset of 0 degrees C road temperatures

Action 2	Pre-salt wet patches minimum of 2hrs before onset of 0 degrees C road temperatures

Action 3	Patrol to determine rain stopping and salt after rain stops but minimum of 3 hrs before onset of 0 degrees C road temperatures

Action 4	Patrol and pre-salt early morning to be complete by 5am

Action 5	Patrol and salt if rain or fog occurs or if signs of frost develop

Action 6	Patrol during period of below 0 degrees C road temperatures

## Notes

Note A

Particular attention should be given to the possibility of water running across carriageways e.g., off adjacent fields after heavy rain, washing salt previously deposited. Such locations should be kept under scrutiny by the patrol system during the forecasted danger period and may require treating individually or further applications to prevent icing from occurring.

Note B

When a road danger warning contains reference to expected hoar frost considerable deposits will usually occur. Hoar frost usually occurs in the early morning and is difficult to cater for because of the probability that any salt deposited on a dry road too soon before its onset may be dispersed before it can become effective. Vigilance is required under these forecasted conditions, which is ideally treated just as the hoar frost is forming. Patrols should be established prior to the forecasted time of onset of road freezing temperatures to enable early application of salt to be deposited if necessary. Hoar frost may be forecast to occur at other times in which case the timing of the salting operations should be adjusted accordingly.

Note C

If under these conditions rain has not ceased by the normal finishing times of the night shift the crews should remain at work until relieved by the day shift crews.

Note D

Under these circumstances rain will freeze on contact with the road surface and full pre-salting must take place even on dry roads. Constant vigilance via the patrol system must be maintained throughout the danger period.

Note E

Warnings are often qualified by altitude in which case differing action may be required for certain areas.

Note F

The possibility of a ‘flash frost’ occurring very suddenly should be considered. This condition requires a combination of circumstances to develop i.e.

    - A calm, clear night

    - At late sunrise (November to January)

    - An increase in traffic at sunrise,

    - Surfaces near the road facing the rising sun e.g., embankments

    - Hazy or misty conditions, maybe with shallow fog banks (although the clear sky must be visible

    - A thin bank of cloud on the south-eastern horizon, which can temporally block the sun (although most of the sky should be clear).

The patrol employees should be instructed to be vigilant of the possibility and if one or more of the circumstances exist those roads most likely to suffer be pre-treated with salt.

## Additional Guidance

The following provides additional guidance relating to the minimum action that must be taken in circumstances where a forecasted state has been issued in the 24Hr Midday Forecast.

Forecast State 	A	Below Freezing	Surfaces Dry

No action on the treatment of roads should be required, however during the period of the forecasted duration of the below freezing road temperatures, a patrol in accordance with the arrangements detailed in the winter maintenance plan should be established.

#### Forecast State 	B

#### Below Freezing	With Hoar Frost Deposits

Treatment of all routes on the pre-salt basis must be undertaken to provide for the complete treatment of all routes prior to the forecasted earliest time of the onset of the frost unless a treatment has been undertaken within the previous 24hrs of the forecasted time of the earliest onset of the frost and no rainfall has been suffered. If treatment has been provided a patrol operation in accordance with the procedure laid down in the winter maintenance plan will be carried out during the period of the forecasted duration of the below freezing road temperatures.

## Forecast State	C	Below Freezing	With Icy Patches

Due to the nature of this forecast consideration must be given to the other factors that are provided with the forecast.

If treatment has been undertaken within the previous 24hrs of the forecasted onset of the road freezing temperatures and no rainfall has been suffered a patrol will be carried out during the period of the forecasted duration of the below freezing road temperatures.

If no treatment has been undertaken or rainfall has been suffered in the previous 24hrs the ‘decision making process’ should be used to identify whether pre-treatment should be taken and any other action that is appropriate to the conditions forecasted for the period concerned.

## Forecast State	D	Widespread Ice

All routes will be treated in good time before the onset of the forecasted freezing road temperatures and any treatment on the previous night will be disregarded. Subsequent patrols will be undertaken to identify the effect of further precipitation on the effect of the treatment undertaken.

## Forecast State	N	No Frost

No action is necessary other than hourly checks of the forecast graph to confirm that no deterioration in the temperature is occurring.

## Forecasted Snowfall

When snowfall is forecasted or lying, a state of continuous alert will be established. Treatments should as a minimum be in accordance with guidance.  Upon any significant amount of snow fall laying the winter maintenance manager will return to Middlefield’s depot and take direct control of the operation.

In general, the procedure for treating of forecasted snowfall is to apply a heavy application of salt prior to or upon the onset of snow falling. Further treatment will be required because of further falls of snow or road temperatures falling below freezing at a level that the previous applications of salt are inadequate to prevent icing. In the worst case where, heavy snowfall is experienced and snowploughing is required, the resources available will be applied on a continuous basis to the main routes to maintain access on the ‘A’ priority routes until such time as they are and remain clear.

## Monitoring of Ice Detection System

Throughout every night in the winter period the ice detection system must be monitored closely, and in particular the night shift charge hand must monitor the forecast graph. Should temperatures fall to below degrees C, the graph and the weather station sites must also be monitored. If circumstances occur where the road temperature begins to fall below the forecasted road temperature, extreme vigilance must be maintained, and the weather station information checked every 15 minutes.

If a deteriorating circumstance is identified the charge hand must instigate making ready the gritting vehicles and instigate treatment if the deterioration continues.

At times outside the period of the night shift operations when a weather forecast identifies the possibility of frost or ice formation two winter maintenance managers will work in cooperation. One will monitor the ice prediction system and the other will carry out patrols as required to confirm actual conditions on the ground.

## Commitment of Manpower Requirements

Part of the decision that the winter maintenance manager will be required to make is the commencement time of the night shift team, the normal attendance at 10pm will be changed if treatment is required before the normal starting time, the number of drivers brought in early will depend upon the requirements as follows:

- Treatment of all routes			Full night shift team

- Patrol					Night shift charge hand (See note 1)

- Monitoring of weather station		Night shift charge hand (See note 1)

Note 1 (the remainder of the night shift team will already be on standby during this period in readiness to attend work immediately if required).

If nightshift drivers are required to attend the depot at 7pm the duty manager will advise them by telephone at 4pm and no later than 5pm.

If the weather is forecast to remain below freezing during the day the duty manager will arrange dayshift drivers from 7am.

On occasion it may be required to bring night shift drivers in before 7pm if sending the dayshift drivers out late would result in them exceeding their driving hours. If this happens the nightshift drivers must be released early to again avoid issues with driver’s hours.

## Procedure for the Issue of Instructions to the Night Shift Charge Hand

Upon arriving at a decision in the light of the forecast the duty manager must issue instructions to the night shift charge hand prior to the commencement of the night shift. These instructions must be entered on the Manager software system but can be supported verbally. The instructions must be clear and identify the action to be taken and the time when the action should commence. Further instruction on the monitoring of the ice detection system in addition to the normal procedures established in the instruction manual for the night shift charge hand should also be recorded on the Manager system. Instruction on the action that must be taken upon completion of the treatment must also be made.

The winter maintenance manager must advise the night shift charge hand of his location and method of availability overnight and particularly where any marginal circumstances are identified, and treatment has not been instructed that the night shift charge hand must use his executive authority to instigate action in the event of conditions deteriorating from the forecasted state.

Procedure upon Weather Updates/Notification of Changing Conditions

Upon notification of a weather update or advice from the night shift charge hand of changing conditions the winter maintenance manager must review his previous instruction. The night shift charge hand has executive authority to act in deteriorating conditions but no powers to curtail treatment in improving conditions and therefore adequate time will be available to make a reasoned decision upon the actions already instigated by the night shift charge hand. Revised instructions should be issued verbally to the night shift charge hand, recorded on the manager’s record sheet, and entered onto the Manager system.

## Documentation

All computer records should be updated daily.

Appendix B

Priority 1 Road Salting Routes

Priority 2 Road Salting Routes

Priority 3 Road Salting Routes

Emergency Road Salting Routes

Appendix C Grit Bin Locations (needs updating)

| Name | Location/Description |
| --- | --- |
| Abingdon Way | At bus stop behind Pizza Hut |
| Albert Road | Opposite 203 Albert Road, Jarrow |
| Anderson Road | Morrisons Loading Area |
| Anderson Street | Next to bus stop by roof top car park |
| Ann Street | On Ann Street at rear of 12/14 Pinewood |
| Ashfield | Adjacent to 2 Fellgate Avenue |
| Ashridge Close | Outside 1 Ashridge Close |
| Bainbridge Avenue | Adjacent to 22 Bainbridge Avenue |
| Bankside Lane | Adjacent to 209 Harton Lane |
| Bardon Court | Opposite 25 Bardon Court |
| Beacon Glade | Adjacent to 102 Beacon Galde |
| Beaconside adjacent to 148 | Adjacent to 148 Beaconside |
| Beaconside adjacent to 192 | Adjacent to 192 Beaconside |
| Beaconside adjacent to 193 | Adjacent to 193 Beaconside |
| Bede Burn Primary School | In the car park |
| Bedford Avenue | In garage area behind block 201-217 bedford Avenue |
| Birch Avenue | Outside 8 Birch Avenue |
| Boldon Library | On footpath between libary and car park |
| Bolingbroke Street | Registry Office Garden |
| Bonsall Court | On Corner at Block 24-30 |
| Brighton Parade Shops (Rear) | In Brighton Parade Car Park |
| Brockley Terrace | Opposite 1 Brockley Terrace |
| Bulmer House | Outside Bulmer House Entrance, Bulmer Road |
| Bulmer Road | Adjacent to 10 Bulmer Road |
| Burns Avenue | Outside 32 Burns Avenue |
| Burrow Street | At entrance to bus stands at South Shields Interchange |
| Calf Close Lane | Adjacent to 2 Pathside |
| Cambridge Road/Marr Avenue | Corner of Cambridge Avenue/Marr Road |
| Cauldwell Avenue | Adjacent to King George Road roundabout |
| Centenary Avenue Shops | At entrance to car park |
| Charlcote Terrace | Adjacent to 1 Charlcote Terrace |
| Charlotte Street | At the junction with Charlotte Street and |
| Chi Metro | Outside Chi Metro Station opposite 327 Laygate |
| Colliery Close | Opposite 1 Colliery Close |
| Corney Street Footbridge | At footbridge entrance |
| Derby Terrace | Opposite 50 Derby Terrace |
| Don Dixon Drive | At entrance to Community building |
| Durham Drive | Opposite 191 Durham Drive |
| Eastway | Outside 12 Eastway |
| Eleanor Street | Adjacent to 27 Eleanor Street |
| Euryalus Court 1 | Outside 10 Euryalus Court |
| Euryalus Court 2 | Outside 21 Euryalus Court |
| Fellgate Avenue 1 | Adjacent to 24 Westfield |
| Fellgate Avenue 2 | Adjacent to 60 Fellgate Avenue |
| Fellgate Bridge | Durham Drive side of Fellgate Bridge |
| Ferry Landing | On ferry ramp |
| Ferry Street | Adjacent to the Alum House |
| Fielding Court Shops Rear | In car Park |
| Fieldway | Adjacent to 132 Fieldway |
| Finchale Road Shops | Next to Finchale Road Shops |
| Flaunden Close | Outside 11 Flaunden Close |
| Fountain Grove | On junction with Fountain Grove and |
| Front Street | Adjacent to 18 Front Street and Boldon Auction House |
| Fulwell Avenue | Outside 76 Fulwell Avenue |
| Galsworthy Road Bus Stop | Bus stop outside of 195 Galsworthy Road |
| Gaskell Avenue Shops Rear | To the rear in car park |
| Glen Street | Outside Glan Primary Care Centre |
| Glenside | In front of 21 Glenside |
| Grays Terrace | On north side of Conation Park |
| Green Lane | To the rear of 320 Green Lane |
| Grotto Gardens | Outside 25 Grotto Gardens |
| Hadrian Road Bridge | Opposite 141 Hadrian Road at entrance to footbridge |
| Hathersage Gardens | On Corner Next to Block 9-15 Bamford Walk |
| Haughton Crescent | Opposite 18 Haughton Crescent |
| Heathway | On the grass opposite the play area |
| Hebburn Health Centre Car Park | In Hebburn Health Centre Car Park (Locked) |
| Hebburn New Town Shops Car Park | In the car park |
| Heddon Way | Metro footpath opposite Dickson's |
| Heddon Way 2 | Outside Glendales building |
| Hedworth Lane | UInder the A194 next to the Robin Hood Pub |
| Hedworth Lane | Opposite 135 Hedworth Lane |
| Hedworth Lane | Aged Miners Homes |
| Hedworth Lane School | Outside Primary School |
| Henderson Road | To the rear of Henderson Road shops |
| Hereford Way | Adjacent to 1 Hereford Way |
| Hereford Way | Opposite 17 Glouster Way |
| Hertford Avenue | Adjacent to 1 Hertford Avenue |
| High Sandgrove | Outside 19 High Sandgrove |
| Highfield Road | Junction with Highfield Road and Cheviot Road |
| Highfield Road Shops | Adjacent to bus stop at shops |
| Hillside | Nect to 1 hillside, behind the wall |
| Holland Park Drive | On grass opposite 15 Hither Green |
| Horsley Hill Police Station | Outside Horsley Hill Police Station |
| Hutton Lane | Hutton Street roundabout |
| Jack Clarke Park Entrance | At entrance to Jack Clarke Park Highfield Road |
| Jarow Bus Station | Sheldon Street at end of bus stand ABC |
| Jarrow Bridge | Wear Sreet at bridge entrance |
| Jarrow Bus Station Car Park | At entrance to car park |
| Jarrow Bus Station Subway | Sheldon Street at the entrance of subway |
| John Clay Street | On the junction with John Clay Street and Chicester Road next to KFC |
| Keppel Street | To the rear of 65 King Street |
| Kingsway | Outside 6 Kingsway |
| Lake Avenue | Adjacent to 101 Lake Avenue |
| Laygate | Outside Kwik Fit garage |
| Laygate Place | Outside entrance to Justice Centre |
| Lichfield Way | Adjacent to 1 Lichfield Way |
| Limecroft | Opposite 11 Limecroft |
| Lizard Lane Turning Bus Circle | At Lizard Lane bus turning circle |
| Lord Nelson Street | Adjacent to Shaftesbury House junction with Whitehead Street |
| Lyons Way | Reed Street Sub station to the rear of48 Elizabeth Diamond Gardens |
| Mansfield Court | Next to 19 Mansfield Court |
| Market Square 1 | Old Town Hall |
| Market Square 2 | Old Town Hall |
| Marlborough Street South | Adjacent to 116 marlborough Street South next to garages |
| Middlefields 1 | Central reserve near to Gatehouse |
| Middlefields 2 | Side of Tyne and Wear Building |
| Middlefields 3 | Opposite Garage Workshops |
| Middlefields 4 | Next to silo entrance |
| Middlefields 5 | Next to silo exit |
| Middlefields 6 | Next to Weighbridge |
| Middlefields 7 | Next to Weighbridge 2 |
| Mile End Road 1 | At junction with River Drive, adjacent to St Aidan and Stephen Church |
| Mile End Road 2 | Adjacent to The Wouldhave pub |
| Mill Dam | Near to the roundabout, opposite the Riverside Bar |
| Mill Grove | Outside 9 Cleadon Hill Road |
| Mill Lane | Adjacent to 1 South View |
| Millais Gardens adjacent to 16 | Outside 16 Millais Gardens |
| Millais Gardens adjacent to 82 | Outside 80 Millais Gardens |
| Monkton Stadium 1 | Behind entrance gates |
| Monkton Stadium 2 | Before main building |
| Monkton Stadium 3 | After main building leading down to playing fields |
| Nevinson Avenue | Outside 32 Nevison Avenue |
| Nevinson Avenue Bus Stop | Adjacetn to bus stop rear of 12 Tarragon Way |
| New George Street Medical Centre | At entrance to medical centre |
| North Marine Park | North Marine Park entrance opposite Fort Street |
| North Road | Junction of North Lane and Cotswold Lane |
| North Road BC | Access Road between shops and club |
| Owen Drive | Outside west Boldon Primary School |
| Page Avenue | On junction with Page Avenue and King George Road |
| Park Road Medical Centre | At bus outside Medical Centre |
| Parkshiel | In layby opposite 40 Parkshiel |
| Percy Street | Rear of St Bede's Church, Westoe Road |
| Perth Avenue Bridge | Outside 1 Perth Avenue at entrance to footbridge |
| Pinfold Court | Outside Pinfold Court, Boldon Lane |
| Portberry Way | Outside 7 Portberry Way |
| Prince Edward Court x2 | Beside the bus stop |
| Prospect Gardens | Outside 10 Prospect Gardens |
| Quarry Lane Shops | Outside Quarry Lane Shops |
| Reading Road | On junction with Mortimer Road and Reading Road |
| Rectory Bank | On rectory Bank |
| Recycling Village 1 | At Recycling Village entrance |
| Recycling Village 2 | At end of Recycling Village |
| Recycling Village 3 | Behind fence at exit of Recycling Village |
| Reed Street | To the rear of 33 Reed Street |
| River Drive | Rear of Wilkinson's |
| Roman Road | Opposite 12 Roman Road at the entrance to the footbridge |
| Saxon Way | Adjacent to Curran House |
| Simonside Hall | Junction of Simonside Hall and Blyton Avenue, opposite 50 Simonside Hall |
| South Dene | Rear of Landreth House |
| South Dene 2 | Rear of Landreth House |
| South Lane | Outside East Boldon Infants School |
| South Shields Crematorium | In car park next to electricity sub station |
| Southlands | Adjaccent to 2 Southlands |
| Spohr Terrace | At the bottom of the zigzag footpath |
| St Davids Way | Outside 2 St Davids Way |
| Stanhope Road | Outside 30 Stanhope Road opposite Needham Court |
| Stanhope Road | Opposite funeral directors |
| Stanhope Road | Behind 497 Stanhope Road at entrance to car park |
| Station Approach Bus Stop | At bus stop opposite Langholme Court |
| Station Approach PROW S17 | On PROW S17 opposite Beggars Bridge |
| Station Road | At the entrance to the Police Station |
| Station Road Cotswold Lane | At junction with Station Road and Cotswold Lane |
| Stirling Avenue Bridge | To the rear of 7 Stirling Avenue at entrance to footbridge |
| Suffolk Gardens | At junction with Lincoln Road |
| Sunniside Farm | Outside Sunniside Farm |
| Sunniside Lane | Outside 31 Sunniside Lane |
| Sunniside Terrace | Outside 16 Sunniside Terrace |
| Swallow Tail Court | In car park |
| Talbot Road | Outside 45 Talbot Road Charles Young Centre |
| Temple Green Outside 12 | Outside 12 Temple Green |
| Temple Green Outside 17 | Outside 17 Temple Green |
| Temple Green Outside 27 | To the rear of 27 Temple Green |
| Thames Avenue | On grass opposite 31 Thames Avenue |
| The Boldon Lad | Adjacent to the Boldon Lad |
| The Britannia | In the car park |
| The Clock Victoria Road East | On footpath next to The Clock at entrance to Windsor Care Home |
| The Cornfields | Opposite 2 The Cornfields |
| The Lonnen adjacent to 136 | Outside 136 The Lonnen |
| The Lonnen Bus Turning Circle | In the car park |
| The Lonnen opposite 152 | Outside 152 The Lonnen |
| The Lonnen opposite 79 | Outside 79 The Lonnen |
| The Vigilant Car Park | At the entrance of The Vigilant Car Park |
| The Willows | Outside 26 The Willows |
| The Willows Jarrow | Opposite 1 The Willows, Jarrow |
| Thorntree Walk | Adjacent to 1 Thorntree Walk |
| Throckley Way Bridge | Throckley Way Footbridge |
| Town Hall | At entrance to car park |
| Trent Drive | Opposite 30 Trent Drive |
| Tyne Dock Metro Station | Hudson Street Tyne Dock Metro Station |
| Ullswater Gardens | Outside 1 Ullswater Gardens |
| Viking Precinct | Adjacent to Morrisons behind the main gate |
| Wark Crescent | Adjacent to 22 Wark Crescent |
| Waterloo Vale | Opposite Subway 6 Waterloo Vale South |
| Wellesley Street | Adjacent to 3 Wellesley Street |
| Wenlock Road | On the junction of Wenlock Road and Blyton Avenue |
| West Park | Entrance to West Park opposite 183 Stanhope Road |
| West Park Road | At junction of Stanhope Road and West Park Road. Opposite Sainsbury's (Ex West park Pub) |
| West Way | At entrance to communal Hall |
| Westoe Rugby Club | In the car park |
| Westoe Village | Southgarth East |
| Wharfedale Drive | On junction of Wharfedale Drive and Dean Road |
| Whitburn Cemetery | In Whitburn Cemetery next to the garage |
| Whitburn Cricket Club | Outside Whitburn Cricket Club |
| Whitburn Library | In Whitburn Library Car Park |
| Whitburn Road PROW | Adjacent to 23 Whitburn Road on PROW B10 |
| Whiteleas Way Shops | Outside 206 Whiteleas Way |
| William Street | At entrance to South Shields Interchange from the car park |
| Winchester Street | To the rear of 75 Fowler Street |
| Windmill Hill | Adjacent to 62 Commerical Road |
| Woodbine Street | Adjacent to 48 Woodbine Street |
| Woodlands Road | Adjacent to 63 Whitburn Road |
| Wye Avenue | Adjacent to 2 Wye Avenue |

Appendix D Weather Station locations

Appendix E Sample Weather Forecast Information

Appendix F Thermal Map

Appendix G Daily Decision record example from Vaisala system

Appendix H The Snow Code

Clearing snow and ice from pavements yourself

There's no law stopping you from clearing snow and ice on the pavement outside your home or from public spaces. It's unlikely you'll be sued or held legally responsible for any injuries on the path if you have cleared it carefully. Follow the snow code when clearing snow and ice safely.

The snow code - tips on clearing snow and ice from pavements or public spaces

Prevent slips

Pay extra attention to clear snow and ice from steps and steep pathways - you might need to use more salt on these areas.

If you clear snow and ice yourself, be careful - don’t make the pathways more dangerous by causing them to refreeze. But don’t be put off clearing paths because you’re afraid someone will get injured.

Remember, people walking on snow and ice have responsibility to be careful themselves. Follow the advice below to make sure you clear the pathway safely and effectively.

Clear the snow or ice early in the day

It’s easier to move fresh, loose snow rather than hard snow that has packed together from people walking on it. So, if possible, start removing the snow and ice in the morning. If you remove the top layer of snow in the morning, any sunshine during the day will help melt any ice beneath. You can then cover the path with salt before nightfall to stop it refreezing overnight.

Use salt or sand - not water

If you use water to melt the snow, it may refreeze and turn to black ice. Black ice increases the risk of injuries, as it is invisible and very slippery. You can prevent black ice by spreading some salt on the area you have cleared. You can use ordinary table or dishwasher salt - a tablespoon for each square metre you clear should work. Don’t use the salt found in salting bins - this will be needed to keep the roads clear.

Be careful not to spread salt on plants or grass as it may cause them damage.

If you don’t have enough salt, you can also use sand or ash. These won’t stop the path icing over as well as salt but will provide good grip under foot.

Take care where you move the snow

When you’re shovelling snow, take care where you put it, so it doesn’t block people’s paths or drains. Make sure you make a path down the middle of the area to be cleared firs, so you have a clear surface to walk on. Then shovel the snow from the centre of the path to the sides.

Offer to clear your neighbours’ paths

If your neighbour will have difficulty getting in and out of their home, offer to clear snow and ice around their property as well. Check that any elderly or disabled neighbours are all right in the cold weather. If you’re worried about them, contact your local council.

Appendix I Weather Domains

Appendix J

# RISK ASSESSMENT--Update

| Directorate Economic Regeneration Asset Management |  |  |  | Section: Engineering |  |  | Assessment No: 013 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ## Work Activity | Loading of Vehicle and Gritting of Roads |  |  |  |  |  |  |  |  |
| Method/Task | Mechanical Gritting of roads within South Tyneside |  |  |  |  |  | Risk rating, Without controls (See Matrix overleaf) |  |  |
| ## Hazards | - Driving/ Manoeuvring of Gritting Vehicle - Loading Operations - Clearing mesh cover - Unauthorised entry into hopper - Traffic - Pedestrians - Accessing areas of vehicle at height - Adverse Weather Conditions - Discharge area - Wet/Contaminated salt - Night Working |  |  |  |  |  | ### High |  |  |
|  |  |  |  |  |  |  | ### Medium |  |  |
|  |  |  |  |  |  |  | ### Low |  |  |
| ## Significant Risks | - Entanglement/ Projectiles at delivery point. - Road Traffic Accident - Violence / Abuse - Slips / Trips / Falls - Entanglement / Projectiles - Fall from height - Eye and skin exposure to salt |  |  |  |  |  |  |  |  |
| Population Exposed | Employees: |  | Sub-Contractors: |  |  | Public: |  |  |  |
|  | Young Persons: |  | #### Special Groups: |  |  | Children: |  |  |  |

| Risk Control Measures | Winter Maintenance Manager directs all gritting operations and determines when Gritters are sent out. Weather conditions will dictate operations. All operators of loading shovel trained and certificated. Area of loading well-lit and general site traffic free Shovel to “Feather” load into hopper to minimise blocking of hopper cage. Any cage blockages to be cleared using rake from ground level or from vehicle platform. Appropriate PPE (eye, skin hand and head protection) to be worn during vehicle loading operations. (Standard described below) Gritting vehicle driven by appropriately qualified driver. All prestart vehicle checks to be carried out by driver and findings recorded. Vehicle defect reporting and repair system in place Driver to wear long sleeved hi-vis clothing at all times Gritting route pre-planned and monitored by Traffic Master. Flashing beacons to be used when vehicle is on highway. Any discharge point blockages to be cleared when vehicle is stationary, and the spinner has been stopped. No person to stand in direct line of discharge point. Communication between vehicle and depot maintained during gritting operations. Hand washing facility and first aid kit on vehicles Welfare facilities available in depot. No plant or vehicles to be operated whilst under the influence of alcohol. Drugs or medication that can affect driving capabilities NO PERSON TO ENTER THE HOPPER AT ANY TIME. Continue on a separate sheet, if necessary |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Instruction Training Required | Training - HGV Licence - Loading Shovel Licence - Reversing Assistants - Gritting Vehicle Training - Health and Safety Awareness |  |  |  |  |  |  |  |
| Personal Protective Equipment | Boot, Steel Toe Caps/ Steal mid-sole (BSEN 345) Eye Protection (BSEN 166 -F) Hearing Protection Ear Plugs EN 352 –2 Earmuffs EN 352 –1 or EN 352 -3 Hand Protection Polychem Red PVC Gauntlets Class III (BSEN 374). Head Protection BSEN 397 High-Vis Jacket (BSEN 471) Class 3 |  |  |  |  |  |  |  |
| Implementation Plan | Salt cover to be installed or salt moved to sheltered area to prevent deterioration and solidification. Platform to be provided to assist with cage Clearing Driver to be issued with torch |  |  | By Whom Manager Manager J McIvor |  | Target Date ASAP 2010 (comp nov) |  |  |
| ## Risk Rating with Controls in place |  |  | ## HIGH | ## MEDIUM |  | ## LOW |  |  |
| Monitoring and Review |  | ##### Review Date |  | By Whom | Target Date |  |  |
|  |  | February 2013 |  |  |  | February 2011 |  |  |
| Action Identified From Review |  |  |  | By Whom |  | Target Date |  |  |
| ## Risk Matrix Likelihood; ###### Worst Case Outco e. Fatality; Major Injury; ## Mi or Injury. ##### Probable; ## HIG; HIGH; MED.. Possible; ## HI H; MED; MED.. Improbable; MED.; LOW; LOW. |
| Assessment conducted by: John McIvor Signed: Date: (Print Name) |  |  |  |  |  |  |  |  |

Appendix K Cosh Assessments for salt

# MATERIAL SAFETY DATA SHEET

  - Identification of the substance/preparation and company

Name of manufacturer:

ICL Limited

Boulby Mine

Loftus

Saltburn by the Sea

Cleveland TS13 4UZ

Telephone +44 (0) 1287 640 140 Fax +44(0) 640 934

Registered in England No GB 915392

PRODUCT NAME:

Rock Salt

APPLICATION:

Road de-icing, Agriculture & Animal feeds ingredient

  - COMPOSITIONAL INFORMATION:

Sodium Chloride NaCl 95%

Synonyms: Rock Salt

  - HAZARD IDENTIFICATION:

Not classified as hazardous under current UK Health, Safety and Environmental legislation when used in the intended application.

  - FIRST AID MEASURES:

Eyes:            Irrigate with plenty of water for 15 minutes.

Seek medical attention if discomfort persists

Skin:             Cover cuts and open wounds

Inhalation:    Seek medical attention if discomfort persists

Ingestion      Seek medical attention

  - FIRE FIGHTING MEASURES:

Flammability: - N/A

Remarks: Gives off irritating fumes when strongly heated.

6. 	ACCIDENTAL RELEASE MEASURES:

Personal precautions wear dust mask if dust is airborne, see Section 8.

Environmental precautions see disposal considerations.

7.   HANDLING AND STORAGE:

Store undercover in a dry area.

8.   EXPOSURE CONTROLS/PERSONAL PROTECTION

Workplace exposure Limits: 8-hour time weighted average

- Total inhalable dust 10mg/m3

- Respirable dust 4g/m3

Engineering controls local exhaust ventilation when excessive dust is produced.

Personal protection suitable dust mask, eye protection and gloves.

PHYSICAL AND CHEMICAL PROPERTIES:

Appearance: 	White to grey crystals containing varying quantities of moisture, 	particle size

range approx. 0.1 – 10.0mm

Melting point Approx. 804oc

Boiling Point approx. 1413oc

Vapour pressure: 1mm at 865oc

Vapour density not applicable

Specific gravity: (dry solid) 2.17

Bulk density (Coarse salt): 1.24T/m3

Bulk density (Fine salt): 1.28T/m3

Solubility in water 35.8gms/100gms water (20oc)

Solubility in water 39.1gms/100gms water (100oc)

- STABILITY AND REACTIVITY:

Corrodes most metal after prolonged contact. Reacts with concentrated sulphuric acid and concentrated nitric acid liberating hydrogen chloride fumes.

- TOXICOLOGICAL INFORMATION:

HEALTH EFFECTS:

On Eyes:	Slight discomfort and stinging.

On Skin:	Stinging if in contact with cuts and open wounds. 			Prolonged contact may cause dermatitis.

By Inhalation: 	Slight.

By Ingestion:	Slight – give plenty of water to drink. Over dosage 			may cause diarrhoea, nausea and vomiting, hyperpnoea, and convulsions.

- ECOLOGICAL INFORMATION:

Short term effects/easily diluted.

- DISPOSAL CONSIDERATIONS:

Can be washed away with large amounts of water/or in dry conditions remove to stock or disposal contractor.

14. TRANSPORT INFORMATION:

Classification for transport: 	Not classified as hazardous for transport

Shipping name: 	Not applicable

UN Number:			Not applicable

ADR/RID:			Not applicable

ICAO/IATO: 			Not applicable

Packaging group: 	Not applicable

Marine pollutant:	     No

15.	REGULATORY INFORMATION:

N/A.

16.	OTHER INFORMATION:

N/A.