# Corporate Health & Safety Manual

3.2 THE CONTROL OF SUBSTANCES HAZARDOUS TO HEALTH (COSHH)

Guidance and Procedures

## Summary

South Tyneside Council have a number of key responsibilities regarding COSHH. These include:

- Control of Substances Hazardous to Health, is the law that requires employers to control substances that are hazardous to health.

- Managers/Supervisor/Team Leader Responsibilities must ensure that they have arrangements in place to access the risks of using substances hazardous to health in respect of their operations.

- Employees Responsibilities are to use all control measures correctly, use within the manner for which they are intended and never interfere, misuse anything provided in the interests of your health and safety.

- Consider controls in the following order, with elimination being the most effective and PPE being the least effective.

## Legal Requirements

The Control of Substances Hazardous to Health Regulations 2002 (as amended).

Regulation 3

Where a duty is placed by these Regulations on an employer in respect of his employees, he shall, so far as is reasonably practicable, be under a like duty in respect of any other person, whether at work or not, who may be affected by the work carried out by the employer except that the duties of the employer.

Regulation 11

Where it is appropriate for the protection of the health of his employees who are, or are liable to be, exposed to a substance hazardous to health, the employer shall ensure that such employees are under suitable health surveillance.

Regulations 10

Ensuring the maintenance of adequate control of the exposure of employees to substances hazardous to health; or for protecting the health of employees, the employer shall ensure that the exposure of employees to substances hazardous to health is monitored in accordance with a suitable procedure.

Regulation 12

Every employer who undertakes work which is liable to expose an employee to a substance hazardous to health shall provide that employee with suitable and sufficient information, instruction and training.

Regulation 13

In order to protect the health of his employees from an accident, incident or emergency related to the presence of a substance hazardous to health at the workplace.

The Health and Safety at Work etc Act 1974 and the Management of Health

and Safety at Work Regulations 1999

Requires South Tyneside Council to provide and maintain a safe working environment, so far as is reasonably practicable.

In addition to the COSHH Regulations 2002, RPE may need to be used to satisfy requirements in the following pieces of legislation. You will need to consider whether any of these Regulations apply to you and comply with any specific requirements they contain on RPE:

- Control of Asbestos Regulations 2012

- Control of Lead at Work Regulations 2002

- Ionising Radiations Regulations 1999

- Confined Spaces Regulations 1997

These Regulations are supported by Approved Codes of Practice. ACOPs give practical guidance on compliance and have a special status in law. If you are prosecuted for a breach of health and safety law, and it is proved that you did not follow the relevant provisions of the code, you will need to show that you have complied with the law in some other way, or a court will find you at fault.

## Important Definitions

COSHH - Control of Substances Hazardous to Health, is the law that requires employers to control substances that are hazardous to health.

Hierarchy of Control - Consider controls in the following order, with elimination being the most effective and PPE being the least effective:

- Elimination – physically remove the hazard

- Substitution – replace the hazard

- Engineering controls – isolate people from the hazard

- Administrative controls – change the way people work

- PPE – protect the worker with equipment

RPE – Where respiratory protective equipment (RPE) is used, it must be able to provide adequate protection for individual wearers. RPE can't protect the wearer if it leaks. A major cause of leaks is poor fit – tight-fitting face masks need to fit the wearer's face to be effective. The wearer of the tight-fitting face mask must be clean shaven.

Adequate – It is right for the hazard and reduces exposure to the level required

to protect the wearer’s health.

Suitable – It is right for the wearer, task and environment, such that the wearer

can work freely and without additional risks due to the RPE.

Identified by the Risk/COSHH Assessment if required to wear a face-fitting mask the employee will be face fit tested at regular intervals. If for medical, religious reasons or for a failed face fit test then alternative protection maybe supplied such as an air fed hood, providing the maximum protection required, ensuring that there is not additional risks such as reduced field of vision, increased exposure due to hood movement.

As people come in all sorts of shapes and sizes it is unlikely that one particular type or size of RPE facepiece will fit everyone. Fit testing will ensure that the equipment selected is suitable for the wearer, this is to be carried out by a trained face fit tester.

A fit test should be repeated whenever there is a change to the RPE type, size, model or material or whenever there is a change to the circumstances of the wearer that could alter the fit of the RPE:

- weight loss or gain

- substantial dental work

- any facial changes (scars, moles, effects of ageing etc) around the face seal area

- facial piercings

- introduction or change in other head-worn personal protective equipment (PPE)

To select RPE that will protect the wearer you will need a basic understanding of:

- the hazardous substance and the amount in the air (exposure)

- the form of the substance in the air (e.g. gas, particle, vapour)

- the type of work being carried out

- any specific wearer requirements, such as other PPE or a need for spectacles.

PPE – Every employer shall ensure that suitable Personal Protective Equipment is provided to his employees who may be exposed to a risk to their health or safety while at work except where and to the extent that such risk has been adequately controlled by other means which are equally or more effective.

You should discard disposable respirators at the end of the shift or sooner if they are heavily contaminated.

LEV – Local Exhaust Ventilation system is used to control or capture airborne hazardous substances and convey them to a point where it is discharged into the atmosphere or safely removed from the system. The extracted air us usually cleaned and then discharged to the atmosphere. LEV systems must be checked regularly and also thoroughly examined and tested by a competent person at least every 14 months. Record all tests, inspections and pre use checks.

Health Surveillance – is any activity which involves obtaining information about employees’ health and which helps protect employees from health risks at work. Health conditions usually develop slowly, by the time symptoms appear irreversible damage has normally been caused. Health conditions can be fatal and include lung cancer, silicosis, Chronic Obstructive Pulmonary Disease (COPD) and asthma.

There are four major routes by which a chemical may enter the body

- Inhalation (breathing)

- Skin (or eye) contact.

- Swallowing (ingestion or eating)

- Injection (skin penetration)

Control measures - Implement control measures around hazardous substances and ensure these are maintained and kept up to date, in full working order and clean where appropriate

Instruction - Provide employees with information, instruction and training around working with hazardous substances

Procedures - Having procedures in place to deal with accidents and emergencies relating to hazardous substances

Assessments - Carry out COSHH/Risk Assessments and review as required

Limits - Ensure the use of hazardous substances doesn't exceed the Workplace Exposure Limit (WEL).

Supervision - Check employees are carrying out tasks as they are supposed to.

Contractors - carrying out work for South Tyneside Council are required to follow the Risk/COSHH Assessments.

Accidents/incidents - Prepare plans and procedures to deal with accidents, incidents and emergencies involving hazard substances, where necessary.

## Procedures

South Tyneside Council as an employer must not undertake any work that is liable to expose any person to a substance hazardous to health, unless a suitable and sufficient risk assessment has been made of the risks to health created by the substances; also the measures necessary to control exposure to it. Exposure to substances hazardous to health must be prevented or where this is not reasonably practicable, controlled to a level that won’t harm health.

Managers/Supervisor/Team Leaders Responsibilities

All departments must ensure that they have arrangements in place to access the risks of using substances hazardous to health in respect of their operations.

Managers/Team Leaders/Supervisors have been either trained across South Tyneside Council to carry out the assessments supported the use of Alcumus Sypol System or by carrying out a COSHH Assessment (Appendix D).

Prevent or control exposure to hazardous substances. The first priority is to eliminate the use of harmful substance or mixture to prevent employees or others being exposed to hazardous substances. Where preventing exposure is not reasonably practicable and hierarchy of control correctly controlled. The eight principles of good practice and hierarchy of control correctly applied should ensure adequate control.

Information regarding the safe use of all substances used by South Tyneside Council employees must be available. Prior to using any substance, ensure you have read the relevant COSHH Assessment and that you have taken all the necessary safety precautions.

Substances hazardous to health include any substance, mixture, dust used at work or arising from workplace activities that is harmful to people’s health in the form in which it occurs in the work activity, including:

- Substances or mixtures classed as toxic, very toxic, harmful, corrosive or irritant (these can be identified by their warning label and the supplier must provide a safety data sheet for them)

- Substances assigned a Workplace Exposure Limit (these can be identified from Safety Data Sheets and are listed in the HSE publication EH40)

- Biological Agents (bacteria and other micro-organisms) if they are directly connected with the work, or if the exposure is incidental to the work (e.g. exposure to bacteria from an air conditioning system that is not properly maintained)

- Any kind of dust if its average concentration in the air exceeds the levels specified in COSHH; and

- Any other substance which creates a risk to health.

Exposure to harmful fumes, vapours, biological contaminants and any dust include silica, wood (hard/soft), and lower toxicity dusts such as gypsum (plaster) etc.

Employees responsibilities

- Read all containers and the COSHH assessment carefully and be aware of the risks involved

- Use all control measures correctly, use within the manner for which they are intended and never interfere, misuse anything provided in the interests of your health and safety

- Use all personal protective equipment and/or clothing where recommended, know how to use the equipment properly and report any defects immediately

- Always store substances correctly e.g. check the temperature and storage with other substances. Do not leave them lying around, where they can create a hazard

- Keep in original containers, do not decant to unlabelled container

- Follow training

- Attend medical checks if you are asked to do so

- Should you experience any health problems when using a substance, report the matter immediately to your Team Leader/Supervisor or Manager

- Take reasonable care for your own health and safety and that of others who may be affected by what you do or do fail to do.

All operatives/site safety health and safety files must have the latest information on COSHH, the assessments are in date. If you notice there is not a COSHH assessment for the substance you

South Tyneside Council Occupational Health Team will

- Arrange and undertake appropriate health surveillance of relevant employees.

- Provide detailed and accurate feedback and analysis on information from assessment on individuals.

- At Pre-Employment health screening is carried out to establish a base line.

- Occupational Health Team send out an appointment letter/email to employee and their manager.

- Health Surveillance is carried out at intervals determined by legislation, council policy, risk assessment and advice from Occupational Health.

- are using, report this to your Team Leader/ Supervisor or Manager.

## Links to Guidance

The Control of Substances Hazardous to health (COSHH) https://www.legislation.gov.uk/uksi/2002/2677/regulation/1/made

The Management of Health and Safety at Work Regulations 1999 https://www.legislation.gov.uk/uksi/1999/3242/regulation/1/made

Health and Safety at Work etc. Act 1974

https://www.legislation.gov.uk/ukpga/1974/37/section/7

Personal Protective Equipment at Work Regulations1992

https://www.legislation.gov.uk/uksi/1992/2966/contents/made

HSE working with Substances Hazardous to Health

https://www.hse.gov.uk/pubns/indg136.pdf

HSE The eight principles of good practice (COSHH – Schedule 2A)

https://www.hse.gov.uk/coshh/detail/goodpractice.htm

HSE COSHH Essentials

https://www.hse.gov.uk/coshh/essentials/index.htm

HSE types of RPE

https://www.hse.gov.uk/respiratory-protective-equipment/types-rpe.htm

Respiratory protective equipment at work- a practical guide

https://www.hse.gov.uk/pubns/priced/hsg53.pdf

HSE all you need to know about disposable masks (FFP3)

https://www.hse.gov.uk/agriculture/dustmasks.htm

EH40/2005 Workplace exposure limits

https://www.hse.gov.uk/pubns/priced/eh40.pdf

Hierarchy of Control

https://www.hse.gov.uk/ppe/managing-risk-using-ppe.htm

Symbol, abbreviation/description of hazard

https://www.hse.gov.uk/chemical-classification/labelling-packaging/hazard-symbols-hazard-pictograms.htm

Risk Phrases

https://www.msds-europe.com/r-phrases-s-phrases/

## Review

These guidance and procedures will be reviewed on an annual basis.

## Document approved by:		H&S Policy Review Group

Last updated/reviewed:		June 2024

Date issued:				June 2024

Date of next review:			August  2025